WASKADA UNIT NO. 1

WATERFLOOD PROGRESS REPORT

January 1, through December 31, 2011

PennWest Exploration

Prepared by:

Siros Mahmody, P.Eng.

Senior Waterflood Exploitation Engineer

TABLE OF CONTENTS

Page

INTRODUCTION                                                                                            3

UNIT HISTORY                                                                                              4

DISCUSSION                                                                                                   6

  - Production Performance

  - Voidage Replacement Ratio Calculation

  - Corrosion and Scale Prevention Program

SUMMARY & RECOMMENDATIONS                                                      7

TABLES                                                                                                            8

  - Table 1 - Rates History

APPENDICES

  - Appendix A – Area Map

  - Appendix B – Production and Injection History plot

  - Appendix C – Voidage replacement Ratio VRR

  - Appendix D – Production and Injection Profiles (Individual wells)

INTRODUCTION

The Waskada Unit No.1 pressure maintenance project commenced water injection into the Mission Canyon designed and in accordance with Manitoba Energy and Mines Approval No. PM 47. (See Appendix A – Area Map)

PRESSURE MAINTENANCE: Governed by Board Order No. PM 47

Unit Information

UNITIZED ZONE: Mission Canyon

Original Unit, June1, 1976 Board Order; Unitization Order No. 22

POOL: Waskada Mission Canyon 3b A (03 42A)

This report documents the performance of the Waskada Unit No.1 pressure maintenance project for the period of January 1 to December 31, 2011.

Unit # 1 is part of main Waskada. The Waskada field is situated on the northeast rim of the Williston Basin in southern Manitoba. It comprises a large portion of Township 1 and 2, Ranges 25 and 26 (W1PM).

The Waskada Fields produce light density crude (approximately 36o API), predominantly from the Lower Amaranth formation.  The interlaminated, shallow marine to subtidal succession of sandstones, siltones, and shale progressively onlaps the Mississippian unconformity surface from basin center, up dip to the north and eastern basin limits in Saskatchewan and Manitoba.  The fine grained reservoir rock has a complex reservoir characterization with 13 to 16 % porosity and permeability on the order of 0.5 to15 md. The lower Amaranth, the oldest Mesozoic unit is a clastic red bed sequence lying directly on the Paleozoic erosional surface. It consists of a series of dolomitic siltstones and sandstones interbedded with argillaceous siltstones and shales. The section is usually subdivided into a lower sandy unit and an overlying shale unit. The lower sequence is the oil production zone. The bulk of pay is founded in the laminated sandstone/siltstone facies.

The Lower Amaranth has been classified into four general lithological types:

- Interbedded shale/siltstone/sandstone by grain size, color and texture

- Siltstone – This lithology occurs in distinct intervals up to two or three meters in thickness. It is generally light green in color and dolomitic.

- Laminated sandstone – This occurs in distinct sandy intervals with a wide range of grain sizes and primary sedimentary structures.

- Massive sandstone – This lithology occurs in thin intervals and usually associated with the laminated sandstones facies. Beds are usually light grey to reddish grey in color and coarse to medium – grained.

UNIT HISTORY

Waskada Unit #1 (Unit History)

| Abbreviated Well ID |  | Date Well Spudded |  | On Prod YYYY/MM | Org Operator Name |  | Ground Elevation (m) |  | TVD (m) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00/03-30-001-25W1/0 |  | 11/21/1967 |  | 1967/12 | Omega Hydcbns Ltd |  | 467 |  | 954 |  |  |  |
| 00/04-30-001-25W1/0 |  | 11/28/1967 |  | 1967/12 | Omega Hydcbns Ltd |  | 469.4 |  | 958.3 |  |  |  |
| 00/05-30-001-25W1/0 |  | 3/16/1967 |  | 1967/05 | Intl Hydcbns Lmtd |  | 467.9 |  | 973.8 |  |  |  |
| 00/06-30-001-25W1/0 |  | 11/6/1967 |  | 1967/12 | Omega Nat Gas Co Ltd |  | 468.2 |  | 952.2 |  |  |  |
| 00/11-30-001-25W1/0 |  | 12/29/1966 |  | 1967/01 | Intl Hydcbns Lmtd |  | 468.2 |  | 957.7 |  |  |  |
| 00/12-30-001-25W1/0 |  | 11/14/1967 |  | 1967/12 | Omega Hydcbns Ltd |  | 467.6 |  | 964.1 |  |  |  |
| 02/09-24-001-26W1/0 |  | 11/24/2011 |  |  |  |  | 466.6 |  | 910.7 |  |  |  |
| 03/09-24-001-26W1/0 |  | 11/1/2011 |  |  |  |  | 468.1 |  | 912.4 |  |  |  |
| 04/09-24-001-26W1/0 |  | 11/12/2011 |  |  |  |  | 468 |  | 909.8 |  |  |  |
| 02/10-24-001-26W1/0 |  | 11/18/2011 |  |  |  |  | 466.9 |  |  |  |  |  |
| 03/10-24-001-26W1/0 |  | 11/7/2011 |  |  |  |  | 467.7 |  | 909.4 |  |  |  |
| 02/12-24-001-26W1/0 |  | 1/9/2010 |  | 2010/05 |  |  | 468 |  | 912 |  |  |  |
| 03/13-24-001-26W1/0 |  | 1/16/2010 |  | 2010/05 |  |  | 467.5 |  | 910.9 |  |  |  |
| 03/16-24-001-26W1/2 |  | 7/8/1994 |  | 2010/11 |  |  | 467.5 |  | 909 |  |  |  |
| 02/11-25-001-26W1/0 |  | 2/14/2011 |  |  |  |  | 465.2 |  | 904.5 |  |  |  |
| 03/11-25-001-26W1/0 |  | 2/6/2011 |  |  |  |  | 465 |  | 906.5 |  |  |  |
| Abbreviated Well ID |  | Date Well Spudded |  | On Prod YYYY/MM | Org Operator Name |  | Ground Elevation (m) |  | TVD (m) |  |  |  |
| 02/14-25-001-26W1/0 |  | 3/7/2011 |  |  |  |  | 465.7 |  | 904.7 |  |  |  |
| 03/14-25-001-26W1/0 |  | 2/28/2011 |  |  |  |  | 465.7 |  | 905.7 |  |  |  |
| 04/14-25-001-26W1/0 |  | 2/21/2011 |  |  |  |  | 465.5 |  | 903.1 |  |  |  |
| Waskada Unit #1 (Production & Injection History) |  |  |  |  |  |  |  |  |  |  |  |  |
| Abbreviated Well ID | First Prod YYYY/MM |  | On Inject. YYYY/MM |  | Last Prod. YYYY/MM | Cumulative OIL Prod. (m3) |  | Cumulative WTR Prod. (m3) |  | First 12 mo. Ave WC% |  | Last Inject. YYYY/MM |
| 00/03-30-001-25W1/0 | 1967/12 |  |  |  | 2011/04 | 22573 |  | 36917 |  | 0 |  |  |
| 00/04-30-001-25W1/0 | 1967/12 |  |  |  | 1991/09 | 23250 |  | 56279 |  | 0 |  |  |
| 00/05-30-001-25W1/0 | 1967/05 |  |  |  | 1977/06 | 15933 |  | 2776 |  | 0 |  |  |
| 00/06-30-001-25W1/0 | 1967/12 |  | 1976/04 |  | 1975/02 | 10286 |  | 0 |  | 0 |  | 1996/04 |
| 00/11-30-001-25W1/0 | 1967/01 |  |  |  | 1980/05 | 6146 |  | 840 |  | 0 |  |  |
| 00/12-30-001-25W1/0 | 1967/12 |  |  |  | 1993/07 | 15552 |  | 13590 |  | 0 |  |  |
| 02/09-24-001-26W1/0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 03/09-24-001-26W1/0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 04/09-24-001-26W1/0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 02/10-24-001-26W1/0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 03/10-24-001-26W1/0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 02/12-24-001-26W1/0 | 2010/05 |  |  |  | 2011/11 | 1152 |  | 925 |  | 43.6 |  |  |
| 03/13-24-001-26W1/0 | 2010/05 |  |  |  | 2011/11 | 3153 |  | 3137 |  | 47.4 |  |  |
| 03/16-24-001-26W1/2 | 2010/11 |  |  |  | 2011/11 | 3844 |  | 5964 |  | 60.5 |  |  |
| 02/11-25-001-26W1/0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 03/11-25-001-26W1/0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 02/14-25-001-26W1/0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 03/14-25-001-26W1/0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 04/14-25-001-26W1/0 |  |  |  |  |  |  |  |  |  |  |  |  |

DISCUSSION

Production Performance

Board Order No. PM 47 provided for pressure maintenance operations in the Waskada Unit No.1. The Unit included an injection wells, 00/06-30-001-25W1/0 (abandoned since 1987/04), and eighteen producers (only 13 of them active now) in the Waskada unit No 1 Pool. Pressure maintenance by water injection in 1976 and continued until 1983. The injector was shut in until 1991. It was put on injection on May 1991, and was shut in again until Jan 1995. It was injecting until March 1996. The injector was abandoned in May 1996. (See Appendix D for oil, water and injection rates).

Voidage Replacement Ratio Calculation:

Upon review of the voidage replacement ratio (VRR) for the Waskada Unit # 1 area, it was shown that the area has been under injected (Cum VRR), although monthly VRR was very high, initially, for few years. This is shown by instantaneous and cumulative VRR for the Waskada Unit # 1 (see Appendix C). Currently there is no active injector in this unit, and PennWest has no plans to re-activate any of the old injectors.

Corrosion and Scale Prevention Program

We currently inject ScalCor down all the new horizontal wells.  Plus, PennWest will be installing cathodic protection on the wells.  Also, the new gathering system is Fiberglass and as such is not susceptible to corrosion.

SUMMARY AND RECOMMENDATIONS

Producers

Current Producing Well

- 02/09-24-001-26W1/0

- 03/09-24-001-26W1/0

- 04/09-24-001-26W1/0

- 02/10-24-001-26W1/0

- 03/10-24-001-26W1/0

- 02/12-24-001-26W1/0

- 03/13-24-001-26W1/0

- 03/16-24-001-26W1/2

- 02/11-25-001-26W1/0

- 03/11-25-001-26W1/0

- 02/14-25-001-26W1/0

- 03/14-25-001-26W1/0

- 04/14-25-001-26W1/0

Current Suspended Producing Wells

None

Abandoned Producing Wells

1. 00/04-30-001-25W1/0 (since 1991/10)

2. 00/05-30-001-25W1/0 (since 1977/07)

3. 00/11-30-001-25W1/0 (since 1980/06)

4. 00/12-30-001-25W1/0 (since 1993/08)

Injectors

Current Injecting Wells

None

Current Suspended Injection Wells

None

Abandoned Injection Wells

1. 00/06-30-001-25W1/0 (since 1996/05)

Currently, there are only one active producers and no injection well in Waskada Unit 1; we do not have any plan for this unit other than monitoring the producer. PennWest’s plan is to concentrate on Lower Amaranth Formation for now.

TABLES

Waskada Unit #1

Table 1: Rates History

| Date | Oil |  | Water |  | Injection Water |  |
| --- | --- | --- | --- | --- | --- | --- |
| Year | m3/year | m3/day | m3/year | m3/day | m3/year | m3/day |
| 1967 | 3,459 | 9.48 | 0 | 0.00 | 0 | 0.00 |
| 1968 | 11,851 | 32.47 | 0 | 0.00 | 0 | 0.00 |
| 1969 | 8,960 | 24.55 | 0 | 0.00 | 0 | 0.00 |
| 1970 | 7,926 | 21.72 | 0 | 0.00 | 0 | 0.00 |
| 1971 | 6,743 | 18.48 | 0 | 0.00 | 0 | 0.00 |
| 1972 | 4,544 | 12.45 | 0 | 0.00 | 0 | 0.00 |
| 1973 | 3,604 | 9.87 | 0 | 0.00 | 0 | 0.00 |
| 1974 | 2,415 | 6.62 | 0 | 0.00 | 0 | 0.00 |
| 1975 | 2,390 | 6.55 | 0 | 0.00 | 0 | 0.00 |
| 1976 | 4,313 | 11.82 | 1,797 | 4.92 | 38,042 | 104.22 |
| 1977 | 6,712 | 18.39 | 3,317 | 9.09 | 12,109 | 33.18 |
| 1978 | 3,359 | 9.20 | 3,969 | 10.87 | 11,609 | 31.81 |
| 1979 | 1,986 | 5.44 | 2,883 | 7.90 | 6,729 | 18.44 |
| 1980 | 1,704 | 4.67 | 4,146 | 11.36 | 8,176 | 22.40 |
| 1981 | 2,014 | 5.52 | 9,351 | 25.62 | 15,005 | 41.11 |
| 1982 | 742 | 2.03 | 6,083 | 16.67 | 14,360 | 39.34 |
| 1983 | 1,190 | 3.26 | 9,346 | 25.61 | 19,474 | 53.35 |
| 1984 | 1,308 | 3.58 | 8,836 | 24.21 | 0 | 0.00 |
| 1985 | 3,056 | 8.37 | 9,208 | 25.23 | 0 | 0.00 |
| 1986 | 2,549 | 6.98 | 11,575 | 31.71 | 0 | 0.00 |
| 1987 | 3,083 | 8.45 | 7,413 | 20.31 | 0 | 0.00 |
| 1988 | 2,537 | 6.95 | 5,102 | 13.98 | 0 | 0.00 |
| Date | Oil |  | Water |  | Injection Water |  |
| Year | m3/year | m3/day | m3/year | m3/day | m3/year | m3/day |
| 1990 | 1,226 | 3.36 | 3,991 | 10.93 | 0 | 0.00 |
| 1991 | 499 | 1.37 | 3,451 | 9.46 | 5,194 | 14.23 |
| 1992 | 322 | 0.88 | 2,626 | 7.19 | 0 | 0.00 |
| 1993 | 584 | 1.60 | 1,016 | 2.78 | 0 | 0.00 |
| 1994 | 384 | 1.05 | 888 | 2.43 | 0 | 0.00 |
| 1995 | 252 | 0.69 | 872 | 2.39 | 1,530 | 4.19 |
| 1996 | 421 | 1.15 | 539 | 1.48 | 215 | 0.59 |
| 1997 | 316 | 0.86 | 935 | 2.56 | 0 | 0.00 |
| 1998 | 15 | 0.04 | 53 | 0.14 | 0 | 0.00 |
| 1999 | 142 | 0.39 | 105 | 0.29 | 0 | 0.00 |
| 2000 | 191 | 0.52 | 330 | 0.90 | 0 | 0.00 |
| 2001 | 357 | 0.98 | 841 | 2.30 | 0 | 0.00 |
| 2002 | 347 | 0.95 | 934 | 2.56 | 0 | 0.00 |
| 2003 | 264 | 0.72 | 835 | 2.29 | 0 | 0.00 |
| 2004 | 66 | 0.18 | 238 | 0.65 | 0 | 0.00 |
| 2005 | 32 | 0.09 | 80 | 0.22 | 0 | 0.00 |
| 2006 | 222 | 0.61 | 920 | 2.52 | 0 | 0.00 |
| 2007 | 74 | 0.20 | 413 | 1.13 | 0 | 0.00 |
| 2008 | 154 | 0.42 | 1,228 | 3.36 | 0 | 0.00 |
| 2009 | 67 | 0.18 | 934 | 2.56 | 0 | 0.00 |
| 2010 | 3284 | 14.17 | 3506 | 13.98 | 0 | 0.00 |
| 2011 | 4944 | 15.68 | 7641 | 24.46 | 0 | 0.00 |

Recent pressure test was performed on 103/13-24-001-26W1/00 on July 2011 and the results of the test are attached to the report.

APPENDIX A

Appendix A – Area Map

APPENDIX B

Appendix B – Production and Injection History plot

APPENDIX C

Appendix C – Voidage replacement Ratio VRR

APPENDIX D

Appendix D – Production and Injection Profiles (Individual wells)