WASKADA LOWER AMARANTH UNIT NO. 1

WATERFLOOD PROGRESS REPORT

January 1, through December 31, 2011

PennWest Exploration

Prepared by:

Siros Mahmody, P.Eng.

Senior Waterflood Exploitation Engineer

TABLE OF CONTENTS

Page

INTRODUCTION                                                                                                          3

UNIT HISTORY                                                                                                             4

DISCUSSION                                                                                                                  7

- Production Performance

- Voidage Replacement Ratio Calculation

- Corrosion and Scale Prevention Program

SUMMARY & RECOMMENDATIONS                                                                     9

TABLES                                                                                                                         13

- Table 1 - Rates History

- Table 2 – Pressure Survey

APPENDICES

- Appendix A – Area Map

- Appendix B – Production and Injection History plot

- Appendix C – Voidage Replacement Ratio VRR

- Appendix D – Production and Injection Profiles (Individual wells)

INTRODUCTION

The WASKADA LOWER AMARANTH UNIT NO.1 pressure maintenance project commenced water injection into the Lower Amaranth designed and in accordance with Manitoba Energy and Mines Approval No. PM 58.

PRESSURE MAINTENANCE: Governed by Board Order No. PM 58

Unit Information:

UNITIZED ZONE: Lower Amaranth

Original Unit, Jan.1, 1983 Board Order; Voluntary

First Enlargement, July. 1, 1983

POOL: Waskada Lower Amaranth A (03 29A)

This report documents the performance of the Waskada Lower Amaranth pressure maintenance project for the period of January 1 to December 31, 2011.

Lower Amaranth Unit # 1 is part of main Waskada. The Waskada field is situated on the northeast rim of the Williston Basin in southern Manitoba. It comprises a large portion of Township 1 and 2, Ranges 25 and 26 (W1PM).

The Waskada Fields produce light density crude (approximately 36o API), predominantly from the Lower Amaranth formation.  The interlaminated, shallow marine to subtidal succession of sandstones, siltones, and shale progressively onlaps the Mississippian unconformity surface from basin center, up dip to the north and eastern basin limits in Saskatchewan and Manitoba.  The fine grained reservoir rock has a complex reservoir characterization with 13 to 16 % porosity and permeability on the order of 0.5 to15 md. The lower Amaranth, the oldest Mesozoic unit is a clastic red bed sequence lying directly on the Paleozoic erosional surface. It consists of a series of dolomitic siltstones and sandstones interbedded with argillaceous siltstones and shales. The section is usually subdivided into a lower sandy unit and an overlying shale unit. The lower sequence is the oil production zone. The bulk of pay is founded in the laminated sandstone/siltstone facies.

The Lower Amaranth has been classified into four general lithological types:

- Interbedded shale/siltstone/sandstone by grain size, color and texture

- Siltstone – This lithology occurs in distinct intervals up to two or three meters in thickness. It is generally light green in color and dolomitic.

- Laminated sandstone – This occurs in distinct sandy intervals with a wide range of grain sizes and primary sedimentary structures.

- Massive sandstone – This lithology occurs in thin intervals and usually associated with the laminated sandstones facies. Beds are usually light grey to reddish grey in color and coarse to medium – grained.

UNIT HISTORY

Waskada Lower Amaranth Unit #1 (Unit History)

| Abbreviated Well ID | Date Well Spudded | On Prod YYYY/MM | Org Operator Name | Ground Elevation (m) | TVD (m) |
| --- | --- | --- | --- | --- | --- |
| 00/09-23-001-26W1/0 | 7/3/1982 | 1982/08 | Omega Hydcbns Ltd | 465.9 | 953.0 |
| 02/10-23-001-26W1/0 | 5/30/1983 | 1983/06 | Omega Hydcbns Ltd | 465.3 | 948.0 |
| 00/15-23-001-26W1/0 | 1/24/1982 | 1982/02 | Omega Hydcbns Ltd | 464.3 | 940.5 |
| 00/16-23-001-26W1/0 | 9/20/1981 | 1981/11 | Omega Hydcbns Ltd | 464.6 | 965.0 |
| 00/09-24-001-26W1/2 | 11/28/1981 | 1983/01 | NCE Petrofund Corp | 468.8 | 944.0 |
| 00/10-24-001-26W1/0 | 2/7/1982 | 1982/03 | Omega Hydcbns Ltd | 466.1 | 935.0 |
| 00/11-24-001-26W1/0 | 2/12/1982 | 1982/03 | Omega Hydcbns Ltd | 465.6 | 935.0 |
| 00/12-24-001-26W1/2 | 11/3/1981 | 1983/02 | NCE Petrofund Corp | 463.9 | 952.0 |
| 02/12-24-001-26W1/0 | 1/9/2010 | 2010/05 |  | 468 | 912.0 |
| 02/13-24-001-26W1/0 | 9/12/1982 |  | Omega Hydcbns Ltd | 465.6 | 951.0 |
| 03/13-24-001-26W1/0 | 1/16/2010 | 2010/05 |  | 467.5 | 910.9 |
| 00/14-24-001-26W1/0 | 11/12/1981 | 1982/02 | Omega Hydcbns Ltd | 465.1 | 951.0 |
| 00/15-24-001-26W1/0 | 9/28/1981 | 1981/10 | Omega Hydcbns Ltd | 466.3 | 961.0 |
| 02/16-24-001-26W1/0 | 2/1/1982 | 1982/02 | Omega Hydcbns Ltd | 468.1 | 956.0 |
| Abbreviated Well ID | Date Well Spudded | On Prod YYYY/MM | Org Operator Name | Ground Elevation (m) | TVD (m) |
| 00/01-25-001-26W1/2 | 7/2/1981 | 1990/05 | NCE Petrofund Corp | 467.3 | 950.0 |
| 02/01-25-001-26W1/0 | 9/12/1982 | 1983/01 | Omega Hydcbns Ltd | 467.2 | 947.0 |
| 00/02-25-001-26W1/2 | 8/22/1981 | 1983/02 | Omega Hydcbns Ltd | 466.1 | 950.0 |
| 00/03-25-001-26W1/2 | 7/14/1981 | 1982/12 | Omega Hydcbns Ltd | 467 | 955.0 |
| 00/04-25-001-26W1/2 | 8/17/1981 | 1982/11 | NCE Petrofund Corp | 466.6 | 963.0 |
| 00/05-25-001-26W1/2 | 8/27/1981 | 1981/10 | NCE Petrofund Corp | 466.9 | 953.0 |
| 00/06-25-001-26W1/0 | 1/17/1982 | 1982/02 | Omega Hydcbns Ltd | 468.1 | 936.0 |
| 00/07-25-001-26W1/0 | 3/1/1982 | 1982/06 | Omega Hydcbns Ltd | 465.7 | 937.0 |
| 02/08-25-001-26W1/0 | 1/11/1982 | 1982/02 | Omega Hydcbns Ltd | 468.4 | 945.0 |
| 00/09-25-001-26W1/0 | 1/4/1982 | 1982/02 | Omega Hydcbns Ltd | 467 | 934.0 |
| 02/09-25-001-26W1/0 | 9/17/1994 | 1994/10 | Omega Hydcbns Ltd | 466.1 | 905.5 |
| 00/10-25-001-26W1/0 | 5/14/1982 | 1982/06 | Omega Hydcbns Ltd | 468.9 | 938.0 |
| 00/11-25-001-26W1/0 | 5/19/1982 | 1982/06 | Omega Hydcbns Ltd | 469.2 | 940.0 |
| 00/12-25-001-26W1/0 | 5/25/1982 | 1982/06 | Omega Hydcbns Ltd | 466.8 | 946.0 |
| 00/13-25-001-26W1/0 | 7/29/1982 | 1982/10 | Omega Hydcbns Ltd | 465.5 | 960.0 |
| 00/14-25-001-26W1/0 | 8/4/1982 | 1982/12 | Omega Hydcbns Ltd | 466.7 | 936.0 |
| 00/15-25-001-26W1/0 | 8/9/1982 | 1982/11 | Omega Hydcbns Ltd | 466.8 | 948.0 |
| 00/16-25-001-26W1/0 | 8/13/1982 | 1982/10 | Omega Hydcbns Ltd | 467.9 | 955.0 |
| B0/16-25-001-26W1/0 | 10/23/1997 | 1997/11 | NCE Rsrcs Grp Inc | 469.7 | 929.0 |
| 00/01-26-001-26W1/2 | 9/14/1981 | 1982/02 | NCE Petrofund Corp | 466.1 | 960.0 |
| 00/02-26-001-26W1/0 | 11/17/1981 | 1982/02 | Omega Hydcbns Ltd | 463.1 | 952.0 |
| 00/07-26-001-26W1/0 | 7/7/1982 | 1982/10 | Omega Hydcbns Ltd | 463.6 | 949.0 |
| 00/08-26-001-26W1/0 | 10/28/1981 | 1982/01 | Omega Hydcbns Ltd | 465.4 | 948.0 |

Waskada Lower Amaranth Unit #1

Production & Injection History

| Abbreviated Well ID | First Prod YYYY/MM | On Inject. YYYY/MM | Last Prod. YYYY/MM | Cumulative OIL Prod. (m3) | Cumulative WTR Prod. (m3) | Last Inject. YYYY/MM |
| --- | --- | --- | --- | --- | --- | --- |
| 00/09-23-001-26W1/0 | 1982/08 |  | 1990/12 | 10,673 | 32,946 |  |
| 02/10-23-001-26W1/0 | 1983/06 |  | 1992/11 | 8,665 | 16,440 |  |
| 00/15-23-001-26W1/0 | 1982/02 | 1983/10 | 1983/09 | 2,958 | 790 | 2004/05 |
| 00/16-23-001-26W1/0 | 1981/11 |  | 2011/11 | 49,400 | 44,524 |  |
| 00/09-24-001-26W1/2 | 1983/01 |  | 2011/08 | 13,547 | 18,353 |  |
| 00/10-24-001-26W1/0 | 1982/03 |  | 2011/11 | 5,065 | 1,626 |  |
| 00/11-24-001-26W1/0 | 1982/03 |  | 1995/10 | 5,111 | 740 |  |
| 00/12-24-001-26W1/2 | 1983/02 |  | 2008/05 | 24,643 | 23,213 |  |
| 02/12-24-001-26W1/0 | 2010/05 |  | 2011/11 | 1,152 | 925 |  |
| 02/13-24-001-26W1/0 |  | 1983/02 |  |  |  | 2006/11 |
| 03/13-24-001-26W1/0 | 2010/05 |  | 2011/11 | 3,153 | 3,137 |  |
| 00/14-24-001-26W1/0 | 1982/02 |  | 2011/11 | 14,311 | 10,363 |  |
| 00/15-24-001-26W1/0 | 1981/10 | 1983/02 | 1983/02 | 3,403 | 431 | 2006/11 |
| 02/16-24-001-26W1/0 | 1982/02 |  | 2002/05 | 12,749 | 18,034 |  |
| 00/01-25-001-26W1/2 | 1990/05 |  | 1990/11 | 0 | 395 |  |
| 02/01-25-001-26W1/0 | 1983/01 |  | 1990/12 | 3,357 | 15,964 |  |
| 00/02-25-001-26W1/2 | 1983/02 |  | 2011/11 | 30,845 | 5,247 |  |
| 00/03-25-001-26W1/2 | 1982/12 |  | 2011/11 | 30,258 | 11,519 |  |
| 00/04-25-001-26W1/2 | 1982/11 |  | 2011/11 | 30,520 | 5,127 |  |
| 00/05-25-001-26W1/2 | 1981/10 | 1983/02 | 1983/02 | 3,741 | 517 | 2011/ 02 |
| 00/06-25-001-26W1/0 | 1982/02 |  | 2010/11 | 45,230 | 66,022 |  |
| 00/07-25-001-26W1/0 | 1982/06 | 1983/02 | 1983/02 | 1,959 | 608 | 2010/11 |
| 02/08-25-001-26W1/0 | 1982/02 |  | 1995/09 | 5,790 | 1,685 |  |
| 00/09-25-001-26W1/0 | 1982/02 |  | 1991/02 | 4,928 | 6,496 |  |
| 02/09-25-001-26W1/0 | 1994/10 |  | 2011/02 | 8,859 | 19,390 |  |
| 00/10-25-001-26W1/0 | 1982/06 |  | 2011/11 | 22,083 | 10,032 |  |
| 00/11-25-001-26W1/0 | 1982/06 |  | 2011/11 | 22,675 | 23,918 |  |
| 00/12-25-001-26W1/0 | 1982/06 |  | 1992/03 | 16,862 | 39,233 |  |
| Abbreviated Well ID | First Prod YYYY/MM | On Inject. YYYY/MM | Last Prod. YYYY/MM | Cumulative OIL Prod. (m3) | Cumulative WTR Prod. (m3) | Last Inject. YYYY/MM |
| 00/13-25-001-26W1/0 | 1982/10 | 1983/12 | 1983/12 | 344 | 242 | 2007/12 |
| 00/14-25-001-26W1/0 | 1982/12 |  | 1989/08 | 439 | 252 |  |
| 00/15-25-001-26W1/0 | 1982/11 | 1983/12 | 1983/09 | 467 | 136 | 2011/02 |
| 00/16-25-001-26W1/0 | 1982/10 |  | 2008/09 | 3,667 | 1,630 |  |
| B0/16-25-001-26W1/0 | 1997/11 |  | 2011/04 | 1,261 | 206 |  |
| 00/01-26-001-26W1/2 | 1982/02 |  | 2011/11 | 32,074 | 27,478 |  |
| 00/02-26-001-26W1/0 | 1982/02 |  | 2011/02 | 26,495 | 21,949 |  |
| 00/07-26-001-26W1/0 | 1982/10 | 1984/01 | 1983/12 | 2,446 | 306 | 2011/05 |
| 00/08-26-001-26W1/0 | 1982/01 |  | 2011/11 | 36,704 | 41,093 |  |

DISCUSSION

Production Performance

Production Response versus Injection: Since injection began, more or less at the same time and the fact that injection rates fluctuated to some degree amongst the injectors, it is difficult to link any production responses to any specific injector. Water breakthrough in certain producers could not be directly correlated with over injection in associated injectors. Some wells showed decrease in oil rate when injection was ceased in 1988-89.

Voidage Replacement Ratio Calculation

With what could be described as very limited success, the waterflood was not maintained properly and injection rate dropped year after year in most cases. The cumulative VRR in the pool is about 1.08 and since there is no active injector in this unit, the current monthly VRR is zero. Although the CUM VRR is greater than one, this can be misleading, from one hand, as the injection across the pool is quite variable, and from other hand it is unknown how much of this water lost to Mission Canyon Formation, located just below the Amaranth Formation. All of the injectors are shut in currently because of formation damage due to fines migration or clay swelling, injection water quality and compatibility with formation water, scale build up or channeling problems (see Appendix C).

To understand the past performance of the Lower Amaranth waterflood, we are doing some reservoir engineering work to come up with potential solutions. One of our plans is to do a pilot program in section 2: The objective of the pilot is to:

- See if we can continuously inject water into the Lower Amaranth Formation  with:-

    - Particle size less than 1 micron

    - Total Suspended Solid (TSS) less than 10 ppm

    - Oil less than 10 ppm

- Inject below the frac pressure

- Test the simulation model that we have built.

2011 Waskada Lower Amaranth Waterflood Pilot Location

The pilot producer will be 102/12-01-02-26W1/00 (a horizontal well) and the injectors will be two vertical wells; 100/12-01-02-26W1 and 100/11-01-02-26 (need to be converted to injectors).

Corrosion and Scale Prevention Program

We currently inject ScalCor down all the new horizontal wells.  Plus, PennWest will be installing cathodic protection on the wells.  Also, the new gathering system is Fiberglass and as such is not susceptible to corrosion.

SUMMARY AND RECOMMENDATIONS

Producers

Current Producing Wells

- 00/16-23-001-26W1/0

- 00/09-24-001-26W1/2

- 00/10-24-001-26W1/0

- 02/12-24-001-26W1/0

- 03/13-24-001-26W1/0

- 00/14-24-001-26W1/0

- 00/02-25-001-26W1/2

- 00/03-25-001-26W1/2

- 02/09-25-001-26W1/0

- 00/10-25-001-26W1/0

- 00/11-25-001-26W1/0

- 00/08-26-001-26W1/0

Current Suspended Wells

- 00/09-24-001-26W1/2	(since	2010/10)

- 00/12-24-001-26W1/2	(since	2008/06)

- 00/04-25-001-26W1/2	(since	2010/12)

- 00/06-25-001-26W1/0	(since	2010/12)

- 00/10-25-001-26W1/0	(since	2010/11)

- 00/16-25-001-26W1/0	(since	2008/10)

Abandoned Wells

- 00/09-23-001-26W1/0	(since	1991/01)

- 02/10-23-001-26W1/0	(since	1992/12)

- 00/11-24-001-26W1/0	(since	1995/11)

- 02/16-24-001-26W1/0	(since	2002/06)

- 00/01-25-001-26W1/2	(since	1990/12)

- 02/01-25-001-26W1/0	(since	1991/01)

- 02/08-25-001-26W1/0	(since	1995/10)

- 00/09-25-001-26W1/0	(since	1991/03)

- 00/12-25-001-26W1/0	(since	1992/04)

- 00/14-25-001-26W1/0      (since 1989/09)

Injectors

Current Injecting Wells

None

Current Suspended Injection Wells

- 00/15-23-001-26W1/0	(since	2004/06)

- 02/13-24-001-26W1/0	(since	2006/12)

- 00/15-24-001-26W1/0	(since	2006/12)

- 00/05-25-001-26W1/2	(since	2011/01)

- 00/07-25-001-26W1/0	(since	2010/12)

- 00/13-25-001-26W1/0	(since	2008/01)

- 00/15-25-001-26W1/0	(since	2010/12)

- 00/07-26-001-26W1/0	(since	2011/01)

(See Appendix E for individual well profile)

Abandoned Wells

None

The behavior of a Lower Amaranth Unit 1 producers are indicated by examining the oil rate versus time plots (see Appendix B). Waskada Lower Amaranth Unit 1 exhibited relatively high initial oil productivity (most of the wells drilled in the past were verticals), rapidly declining to flat/low decline rates, with almost no discernible water flood response.  This behavior can be explained by drop in the reservoir pressure from initial (approximately 8700 kPag) to above in some wells or below in others bubble point pressure (about 4200 kPag) followed by solution gas breakout which adversely affected the relative permeability to oil. (see Table # 2: Pressure Surveys)

Also, it is believed that fracture stimulation treatments, performed on these wells prior to initiation of water injection, “broke” through into the higher productivity Mississippian zone and that majority of injected water to date has entered this zone. This is one of the major explanations for lack of waterflood response to date and the continued decline in oil productivities.

The Waskada Lower Amaranth is becoming a non-conventional tight oil resources play that utilizes horizontal multi-stage frac drilling technology (small multi-stage frac stimulations on newly drilled wells will remain “in zone” within the Lower Amaranth) to re-develop the thick low perm oil zones adjacent to the conventional Amaranth zone that was discovered in the 1980’s. PennWest drilled 3 horizontal wells, to increase the Recovery Factor (RF), in year 2010, 102/12-24-001-26W1, 103/13-24-001-26W1 and 103/16-24-001-26W1/00. Penn West’s follow up plan once we have drilled more horizontal wells in the Units  is to convert some of the recent horizontal producing wells to injection wells to increase the sweep efficiency and ultimately increase the recoverable oil in place.

The following is the HZ Multi Stage Fracture Technology that we are using in our new development

TABLES

Waskada Lower Amaranth Unit #1

Table 1: Rates History

| Date | Oil |  | Water |  | Injection Water |  |
| --- | --- | --- | --- | --- | --- | --- |
| Year | m3/year | m3/day | m3/year | m3/day | m3/year | m3/day |
| 1981 | 1,293 | 3.54 | 647 | 1.77 | 0 | 0.00 |
| 1982 | 34,589 | 94.76 | 8,564 | 23.46 | 0 | 0.00 |
| 1983 | 63,455 | 173.85 | 26,801 | 73.43 | 98,755 | 270.56 |
| 1984 | 55,295 | 151.49 | 28,617 | 78.40 | 132,955 | 364.26 |
| 1985 | 41,787 | 114.48 | 27,555 | 75.49 | 86,897 | 238.07 |
| 1986 | 37,496 | 102.73 | 28,006 | 76.73 | 107,802 | 295.35 |
| 1987 | 32,268 | 88.41 | 39,340 | 107.78 | 82,230 | 225.29 |
| 1988 | 28,003 | 76.72 | 25,126 | 68.84 | 64,867 | 177.72 |
| 1989 | 23,117 | 63.33 | 20,406 | 55.91 | 9,366 | 25.66 |
| 1990 | 19,472 | 53.35 | 16,000 | 43.83 | 24,156 | 66.18 |
| 1991 | 13,902 | 38.09 | 16,050 | 43.97 | 31,802 | 87.13 |
| 1992 | 12,342 | 33.81 | 17,729 | 48.57 | 41,247 | 113.01 |
| 1993 | 10,180 | 27.89 | 20,905 | 57.28 | 81,517 | 223.33 |
| 1994 | 11,592 | 31.76 | 15,873 | 43.49 | 26,062 | 71.40 |
| 1995 | 11,593 | 31.76 | 18,340 | 50.25 | 56,183 | 153.92 |
| 1996 | 11,489 | 31.48 | 16,992 | 46.55 | 57,114 | 156.48 |
| 1997 | 8,731 | 23.92 | 15,108 | 41.39 | 46,833 | 128.31 |
| 1998 | 7,120 | 19.51 | 13,147 | 36.02 | 24,871 | 68.14 |
| 1999 | 5,829 | 15.97 | 11,053 | 30.28 | 17,130 | 46.93 |
| 2000 | 5,998 | 16.43 | 11,740 | 32.16 | 12,856 | 35.22 |
| 2001 | 6,356 | 17.41 | 10,508 | 28.79 | 15,584 | 42.70 |
| 2002 | 5,846 | 16.02 | 9,016 | 24.70 | 12,815 | 35.11 |
| 2003 | 5,399 | 14.79 | 7,476 | 20.48 | 10,947 | 29.99 |
| 2004 | 4,779 | 13.09 | 6,511 | 17.84 | 9,381 | 25.70 |
| 2005 | 4,146 | 11.36 | 7,149 | 19.59 | 8,353 | 22.88 |
| 2006 | 4,094 | 11.22 | 7,752 | 21.24 | 11,382 | 31.18 |
| 2007 | 3,775 | 10.34 | 8,308 | 22.76 | 36,593 | 100.26 |
| 2008 | 3,227 | 8.84 | 8,271 | 22.66 | 10,750 | 29.45 |
| 2009 | 4,690 | 12.85 | 10,887 | 29.83 | 6,697 | 18.35 |
| 2010 | 5,310 | 14.55 | 10,566 | 28.95 | 17,244 | 47.24 |
| 2011 | 2,664 | 8.80 | 6,526 | 17.88 | 119 | 0.32 |

Waskada Lower Amaranth Unit #1

Table 2: Pressure Surveys

| Location | Shut In Date | Date of Survey | Type of Survey | Pressure @ Datum Depth (kPa) |
| --- | --- | --- | --- | --- |
| 02/12-24-001-26W1/0 | 17-Oct-10 | 24-Oct-10 | BHP Build Up | 1016 |
| 03/13-24-001-26W1/0 | 17-Oct-10 | 24-Oct-10 | BHP Build Up | 3179 |
| 00/15-24-001-26W1/0 | Dec-89 | (18 days) | Static Gradient | 10482 |
| 00/06-25-001-26W1/0 | (8 days) | 11-Dec-06 | Acoustic Build Up | 4186 |
| 00/09-25-001-26W1/0 |  | 2008 | BHP, Assuming WC from Last Prod'n | 4881 |
| 02/09-25-001-26W1/0 | 17-Oct-10 | 24-Oct-10 | BHP Build Up | 1743 |
| 00/15-25-001-26W1/0 | Jan-90 | (68 days) | Static Gradient | 11144 |
| 00/16-25-001-26W1/0 |  | 2008 | BHP, Assuming WC from Last Prod'n | 5765 |
| 00/08-26-001-26W1/0 |  | 2008 | BHP, Assuming WC from Last Prod'n | 6267 |

APPENDIX A

Appendix A – Area Map

APPENDIX B

Appendix B – Production and Injection History plot

APPENDIX C

Appendix C – Voidage Replacement Ratio VRR

APPENDIX D

Appendix D – Production and Injection Profiles (Individual Wells)

.