| Petroleum Branch 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 T 204-945-6577 F 204-945-0586 www.manitoba.ca |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |
| (January 1/19 – January 7/19) |  |  |  |  |  |  |  |
| DRILLING ACTIVITY | To January 7/19 |  | To January 8/18 |  |  |  | 2018 Total |
| Drilling Licences Issued | 5 |  | 0 |  |  |  | 290 |
| Licences Cancelled | 0 |  | 0 |  |  |  | 13 |
| Vertical Wells Drilled | 1 |  | 0 |  |  |  | 13 |
| Horizontal Wells Drilled | 3 |  | 4 |  |  |  | 263 |
| Stratigraphic Test Holes Drilled | 0 |  | 0 |  |  |  | 0 |
| Wells Drilled - Total | 4 |  | 4 |  |  |  | 276 |
| No. of Metres Drilled | 7 326 |  | 7 858 |  |  |  | 586 264 |
| Wells Re-entered | 0 |  | 0 |  |  |  | 0 |
| Wells Being Drilled | 6 |  | 7 |  |  |  | 0 |
| No. of Active Rigs | 6 |  | 7 |  |  |  | 8 |
| Wells Licenced but Not Spudded | 70 |  | 63 |  |  |  | 74 |
| Wells Completed as Potential Oil Wells | 3 |  | 4 |  |  |  | 271 |
| Wells Abandoned Dry | 0 |  | 0 |  |  |  | 3 |
| Wells Drilled but Not Completed | 0 |  | 0 |  |  |  | 0 |
| Other Completions | 1 |  | 0 |  |  |  | 2 |
| New Wells on Production | 0 |  | 0 |  |  |  | 85 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |
| Geophysical Programs Licenced | 0 |  | 0 |  |  |  | 3 |
| Licences Cancelled | 0 |  | 0 |  |  |  | 1 |
| Kilometers Licenced | 0 |  | 0 |  |  |  | 82 |
| Kilometers Run | 0 |  | 0 |  |  |  | 638 |
|  | 2018 |  | 2017 |  |  |  |  |
| OIL PRICES (Average) | $/m3 ($/bbl) |  | $/m3 ($/bbl) |  |  |  |  |
| Month of November | 347.77 (55.26) |  | 447.77 (71.15) |  |  |  |  |
| Month of December | 259.97 (41.31) |  | 440.28 (69.96) |  |  |  |  |
| OIL PRODUCTION (M3) | 2018 |  | 2017 |  |  |  |  |
| Month of June | 190 973.9 |  | 178 533.1 |  |  |  |  |
| Month of July | 189 452.4 |  | 185 461.4 |  |  |  |  |
| Month of August | 189 804.4 |  | 193 117.8 |  |  |  |  |

WEEKLY WELL ACTIVITY REPORT

07 January 2019

| Lic. No.: 7289 | 1885683 Pierson HZNTL 4-24-2-29 (WPM) UWI: 100.04-24-002-29W1.00 – Leg #1 UWI: 100.04-24-002-29W1.02 – Leg #2 UWI: 102.03-24-002-29W1.03 – Leg #3 Revised UWI: 102.03-24-002-29W1.04 – Leg #4 Revised Please note revised UWI’s Leg #3 & #4 to reflect As Drilled: UWI: 102.03-24-002-29W1.03 – Leg #3 UWI licenced as 100.04-24… changed to 102.03-24… |
| --- | --- |
|  | UWI: 102.03-24-002-29W1.04 – Leg #4 UWI licenced as 100.04-24… changed to 102.03-24… |
| Lic. No.: 10922 | Ewart Unit No. 12 Prov. HZNTL A9-20-8-28 (WPM) UWI: 102.09-20-008-28W1.00 Revised Please note revised UWI and Well Name to reflect As Drilled: UWI licenced as 102.08-20… changed to 102.09-20… Well Name licenced as A8-20…changed to A9-20-8-28 |
| Lic. No.: 10994 | Corex Virden HZNTL 10-5-10-25 (WPM) UWI: 100.10-05-010-25W1.00 - Revised Please note revised UWI and Well Name to reflect As Drilled: UWI licenced as 100.15-05… changed to 100.10-05… Well Name licenced as 15-5…changed to 10-5-10-25 |
| Lic. No.: 11024 | Melita Pierson Prov. HZNTL 12-28-2-28 (WPM) UWI: 100.12-28-002-28W1.00 – Leg #1 Revised UWI: 100.12-28-002-28W1.02 – Leg #2 Revised Please note revised UWI’s and Name Change to reflect As Drilled: UWI: 100.12-28-002-28W1.00 – Leg #1 |
|  | UWI licenced as 100.13-28… changed to 100.12-28… UWI: 100.12-28-002-28W1.02 – Leg #2 UWI licenced as 100.13-28… changed to 100.12-28… Well Name licenced as 13-28… changed to 12-28-2-28 |
| Lic. No.: 11035 | East Manson Unit No. 3 HZNTL 15-21-13-28 (WPM) UWI: 100.15-21-013-28W1.00 Revised Please note revised UWI and Name change to reflect As Drilled: UWI licenced as 102.16-21…changed to 100.15-21… Well Name licenced as A16-21… changed to 15-21-13-28 |
| Lic. No.: 11051 | Waskada Unit No. 3 HZNTL 14-32-1-25 (WPM) UWI: 102.14-32-001-25W1.00 |
|  | Spud Date: 03-Jan-2019 K.B. Elevation: 475.51 m Surface Casing: 219.10 mm @ 138.00 m with 10.0 t Finished Drilling: 05-Jan-2019 Total Depth: 1572.00 m Intermediate Casing: 139.70 mm @ 1572.00 m with 26.5 t Rig Released: 05-Jan-2019 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 05-Jan-2019 |
| Lic. No.: 11055 | North Virden Scallion Unit No. 1 Prov. HZNTL A14-24-11-26 (WPM) UWI: 104.14-24-011-26W1.00 - Leg #1 UWI: 104.14-24-011-26W1.02 - Leg #2 Status: Completing (COMP) Completing: 07-Jan-2019 |
| Lic. No.: 11059 | Waskada Unit No. 3 HZNTL A14-32-1-25 (WPM) UWI: 103.14-32-001-25W1.00 |
|  | Spud Date: 06-Jan-2019 K.B. Elevation: 475.61 m Surface Casing: 219.10 mm @ 135.00 m with 6.5 t Status: Drilling Ahead (DR) Drilling Ahead: 07-Jan-2019 |
| Lic. No.: 11087 | North Virden Scallion Unit No. 1 HZNTL 6-21-11-26 (WPM) UWI: 102.06-21-011-26W1.00 Spud Date: 07-Jan-2019 K.B. Elevation: 463.53 m |
|  | Status: Running Casing (CSG) Running Casing: 07-Jan-2019 |
| Lic. No.: 11113 | Tundra Daly Sinclair SWD 5-6-11-28 (WPM) UWI: 100.05-06-011-28W1.00 Finished Drilling: 03-Jan-2019 Total Depth: 1774.00 m Rig Released: 05-Jan-2019 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 05-Jan-2019 |
| Lic. No.: 11115 | South Pierson Unit No. 1 Prov. HZNTL 13-16-2-29 (WPM) UWI: 103.13-16-002-29W1.00 Spud Date: 04-Jan-2019 K.B. Elevation: 482.04 m Surface Casing: 244.50 mm @ 169.00 m with 13.0 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 07-Jan-2019 |
| Lic. No.: 11122 | Corex Virden HZNTL 14-3-12-26 (WPM) UWI: 100.14-03-012-26W1.00 Spud Date: 03-Jan-2019 K.B. Elevation: 464.52 m Surface Casing: 244.50 mm @ 199.00 m with 12.0 t Intermediate Casing: 177.80 mm @ 770.00 m with 11.3 t Finished Drilling: 06-Jan-2019 Total Depth: 1786.00 m Rig Released: 07-Jan-2019 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 07-Jan-2019 |
| Lic. No.: 11130 | Tundra Daly Sinclair HZNTL B1-4-9-28 (WPM) UWI: 103.01-04-009-28W1.00 Status: Completing (COMP) Completing: 07-Jan-2019 |
| Lic. No.: 11132 | Tundra Daly Sinclair HZNTL B1-1-9-29 (WPM) UWI: 103.01-01-009-29W1.00 Status: Completing (COMP) Completing: 04-Jan-2019 |
| Lic. No.: 11135 | Tundra Daly Sinclair Prov. HZNTL C5-36-8-29 (WPM) UWI: 104.05-36-008-29W1.00 Status: Completing (COMP) Completing: 05-Jan-2019 |
| Lic. No.: 11145 | Tundra Daly Sinclair HZNTL B16-28-8-28 (WPM) UWI: 103.16-28-008-28W1.00 Please note revision to Report dated 17-Dec-2018 Drilling Contractor: Trinidad Drilling Ltd. – Rig #10 Spud Date: 05-Jan-2019 K.B. Elevation: 500.13 m Surface Casing: 244.50 mm @ 138.00 m with 7.5 t Status: Drilling Ahead (DR) Drilling Ahead: 07-Jan-2019 |
| Lic. No.: 11150 | Sinclair Unit No. 19 HZNTL C13-25-8-29 (WPM) UWI: 104.13-25-008-29W1.00 Spud Date: 06-Jan-2019 K.B. Elevation: 517.90 m Surface Casing: 244.50 mm @ 137.00 m with 7.5 t Status: Drilling Ahead (DR) Drilling Ahead: 07-Jan-2019 |
| Lic. No.: 11151 | Tundra Daly Sinclair HZNTL B1-15-9-29 (WPM) UWI: 103.01-15-009-29W1.00 Spud Date: 03-Jan-2019 K.B. Elevation: 536.45 m Surface Casing: 244.50 mm @ 135.00 m with 7.5 t Finished Drilling: 05-Jan-2019 Total Depth: 2194.00 m Intermediate Casing: 139.70 mm @ 2194.00 m with 35.0 t Rig Released: 06-Jan-2019 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 06-Jan-2019 |
| Lic. No.: 11153 | Corex Daly Sinclair HZNTL 8-31-9-28 (WPM) UWI: 102.08-31-009-28W1.00 Spud Date: 03-Jan-2019 K.B. Elevation: 525.23 m Surface Casing: 244.50 mm @ 150.00 m with 9.5 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 07-Jan-2019 |
| Lic. No.: 11175 | Waskada Unit No. 19 HZNTL B1-34-1-25 (WPM) UWI:104.01-34-001-25W1.00 Licence Issued: 07-Jan-2019 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Trinidad Drilling Ltd. - Rig# 12 Surface Location: 6A-34-1-25 Co-ords: 447.66 m N of S of Sec 34 697.25 m E of W of Sec 34 Grd Elev: 472.18 m Proj. TD: 1696.20 m (Triassic) Field: Waskada Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11176 | Waskada Unit No. 19 Prov. HZNTL B16-34-1-25 (WPM) UWI:104.16-34-001-25W1.00 Licence Issued: 07-Jan-2019 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Trinidad Drilling Ltd. - Rig# 12 Surface Location: 14A-34-1-25 Co-ords: 406.57 m S of N of Sec 34 689.65 m E of W of Sec 34 Grd Elev: 472.98 m Proj. TD: 1705.10 m (Triassic) Field: Waskada Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11177 | Corval Daly Sinclair HZNTL C5-27-8-28 (WPM) UWI:105.05-27-008-28W1.00 Licence Issued: 07-Jan-2019 Licensee: Corval Energy Ltd. Mineral Rights: Corval Energy Ltd. Contractor: Precision Drilling - Rig# 156 Surface Location: 3B-34-8-28 Co-ords: 120.24 m N of S of Sec 34 423.33 m E of W of Sec 34 Grd Elev: 477.25 m Proj. TD: 1731.00 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11178 | Corval Daly Sinclair HZNTL A6-27-8-28 (WPM) UWI:103.06-27-008-28W1.00 Licence Issued: 07-Jan-2019 Licensee: Corval Energy Ltd. Mineral Rights: Corval Energy Ltd. Contractor: Precision Drilling - Rig# 156 Surface Location: 3B-34-8-28 Co-ords: 120.24 m N of S of Sec 34 443.33 m E of W of Sec 34 Grd Elev: 477.02 m Proj. TD: 1738.00 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11179 | Sinclair Unit No. 3 Prov. HZNTL A5-11-8-29 (WPM) UWI:103.05-11-008-29W1.00 Licence Issued: 07-Jan-2019 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Trinidad Drilling Ltd. - Rig# 9 Surface Location: 5B-12-8-29 Co-ords: 577.91 m N of S of Sec 12 196.84 m E of W of Sec 12 Grd Elev: 503.62 m Proj. TD: 2529.00 m (Devonian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |

NEW UNIT APPROVALS

RE: South Pierson Unit No. 4 Project Approval

The effective date for South Pierson Unit No. 4 is January 1, 2019 covering the following in the Pierson Field:

| LSDs | Section | Township | Range |
| --- | --- | --- | --- |
| 9-16 | 14 | 2 | 29 |

Field pool code: 7 35A

RE: Ewart Unit No. 15 Project Approval

The effective date for Ewart Unit No. 15 is January 1, 2019 covering the following in the Daily Sinclair Field:

| LSDs | Section | Township | Range |
| --- | --- | --- | --- |
| 9-16 | 33 | 8 | 28 |

Field pool code: 1 59A