| Natural Resources & Northern Development Petroleum Branch 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 T 204-945-6577 F 204-945-0586 www.manitoba.ca |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |
| (January 1/22 – January 31/22) |  |  |  |  |  |  |  |
| DRILLING ACTIVITY | To Jan 31/22 |  | To February 1/21 |  |  |  | 2021 Total |
| Drilling Licences Issued | 9 |  | 4 |  |  |  | 178 |
| Licences Cancelled | 0 |  | 0 |  |  |  | 9 |
| Vertical Wells Drilled | 1 |  | 0 |  |  |  | 9 |
| Horizontal Wells Drilled | 25 |  | 20 |  |  |  | 164 |
| Stratigraphic Test Holes Drilled | 0 |  | 0 |  |  |  | 1 |
| Wells Drilled - Total | 26 |  | 20 |  |  |  | 174 |
| No. of Metres Drilled | 55 007 |  | 44 548 |  |  |  | 372 739 |
| Wells Re-entered | 1 |  | 0 |  |  |  | 6 |
| Wells Being Drilled | 6 |  | 2 |  |  |  | 0 |
| No. of Active Rigs | 6 |  | 2 |  |  |  | 0 |
| Wells Licenced but Not Spudded | -19 |  | 37 |  |  |  | 62 |
| Wells Completed as Potential Oil Wells | 25 |  | 20 |  |  |  | 168 |
| Wells Abandoned Dry | 0 |  | 0 |  |  |  | 3 |
| Wells Drilled but Not Completed | 0 |  | 0 |  |  |  | 0 |
| Other Completions | 1 |  | 0 |  |  |  | 3 |
| New Wells on Production | 0 |  | 1 |  |  |  | 34 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |
| Geophysical Programs Licenced | 0 |  | 0 |  |  |  | 6 |
| Licences Cancelled | 0 |  | 0 |  |  |  | 0 |
| Kilometers Licenced | 0 |  | 0 |  |  |  | 439 |
| Kilometers Run | 0 |  | 0 |  |  |  | 438 |
|  | 2021 |  | 2020 |  |  |  |  |
| OIL PRICES (Average) | $/m3 ($/bbl) |  | $/m3 ($/bbl) |  |  |  |  |
| Month of November | 43 615.98 (97.88) |  | 303.27 (48.19) |  |  |  |  |
| Month of December | 43 615.98 (97.88) |  | 346.19 (55.01) |  |  |  |  |
| OIL PRODUCTION (M3) | 2021 |  | 2020 |  |  |  |  |
| Month of December | 190 750.2 |  | 179 854.0 |  |  |  |  |

WEEKLY WELL ACTIVITY REPORT

31 January 2022

| Lic. No.: 11630 | Tundra Daly Sinclair HZNTL B8-7-10-28 (WPM) UWI: 104.08-07-010-28W1.00 Status: Completing (COMP) Completing: 25-Jan-2022 |
| --- | --- |
| Lic. No.: 11631 | Tundra Daly Sinclair HZNTL 8-8-10-29 (WPM) UWI: 102.08-08-010-29W1.00 Status: Completing (COMP) Completing: 30-Jan-2022 |
| Lic. No.: 11637 | Tundra Daly Sinclair HZNTL 9-32-10-28 (WPM) 102.09-32-010-28W1.00 Status: Completing (COMP) Completing 28-Jan-2022 |
| Lic. No.: 11644 | Tundra Daly Sinclair Prov HZNTL 1-24-10-29 (WPM) UWI: 102.01-24-010-29W1.00 Status: Completing (COMP) Completing: 31-Jan-2022 |
| Lic. No.: 11636 | Sundog Daly Sinclair HZNTL 9-7-9-27 (WPM) UWI: 100.09-07-009-27W1.00 Please note revision to Report dated 06-Dec-2021 Drilling Contractor: Stampede Drilling Inc. – Rig #3 Spud Date: 29-Jan-2022 K.B. Elevation: 488.12 m Surface Casing: 244.50 mm @ 166.00 m with17.0 t Status: Drilling Ahead (DR) Drilling Ahead: 31-Jan-2022 |
| Lic. No.: 11647 | Tundra Daly Sinclair Prov. HZNTL C9-8-9-29 (WPM) UWI: 104.09-08-009-29W1.00 Please note revision to Report dated 13-Dec-2021 Drilling Contractor: Ensign Drilling Inc. – Rig #10 Spud Date: 28-Jan-2022 K.B. Elevation: 544.26 m Surface Casing: 244.50 mm @ 135.00 m with 8.0 t Status: Drilling Ahead (DR) Drilling Ahead: 31-Jan-2022 |
| Lic. No.: 11650 | Tundra Daly Sinclair Prov. HZNTL C1-30-8-28 (WPM) UWI: 104.01-30-008-28W1.00 Spud Date: 30-Jan-2022 K.B. Elevation: 516.32 m Status: Drilling Ahead (DR) Drilling Ahead: 31-Jan-2022 |
| Lic. No.: 11651 | Tundra Daly Sinclair Prov. HZNTL B1-6-9-28 (WPM) UWI: 103.01-06-009-28W1.00 Spud Date: 25-Jan-2022 K.B. Elevation: 516.66 m Surface Casing: 244.50 mm @ 135.00 m with 6.0 t Finished Drilling: 29-Jan-2022 Total Depth: 2296.00 m Production Casing: 139.70 mm @ 2296.00 m with 32.5 t Rig Released: 30-Jan-2021 Status: Waiting on Service Rig (WOSR) Waiting on Service Rig: 30-Jan-2022 |
| Lic. No.: 11653 | Ebor Unit No. 4 HZNTL C16-5-9-29 (WPM) UWI: 104.16-05-009-29W1.00 Finished Drilling: 27-Jan-2022 Total Depth: 2223.00 m Production Casing: 139.70 mm @ 2223.00 m with 33.5 t Rig Released: 27-Jan-2022 Status: Waiting on Service Rig (WOSR) Waiting on Service Rig: 27-Jan-2022 |
| Lic. No.: 11657 | Tundra Daly Sinclair HZNTL 1-22-10-28 (WPM) UWI: 100.01-22-010-28W1.00 Spud Date: 25-Jan-2022 K.B. Elevation: 513.75 m Surface Casing: 244.50 mm @ 136.00 m with 7.0 t Finished Drilling: 28-Jan-2022 Total Depth: 2255.00 m Production Casing: 139.70 mm @ 2255.00 m with 32.0 t Rig Released: 29-Jan-2022 Status: Waiting On Service Rig (WOSR) Waiting on Service Rig: 29-Jan-2022 |
| Lic. No.: 11659 | BCX Pierson SWD 4-7-3-29 (WPM) UWI: 100.04-07-003-29W1.00 Production Casing: 177.80 mm @ 1071.00 m with 19.0 t Rig Released: 25-Jan-2022 Status: Waiting On Service Rig (WOSR) Waiting on Service Rig: 25-Jan-2022 |
| Lic. No.: 11661 | BCX Pierson HZNTL 2-31-10-25 (WPM) UWI: 100.02-31-010-35W1.00 Spud Date: 26-Jan-2022 K.B. Elevation: 491.60 m Surface Casing: 244.50 mm @ 145.00 m with 12.5 t Intermediate Casing: 177.80 mm @ 1157.00 m with 19.5 t Status: Drilling Ahead (DR) Drilling Ahead 31-Jan-2022 |
| Lic. No.: 11666 | Corex Virden HZNTL B2-31-10-25 (WPM) UWI: 100.02-31-010-25W1.00 Intermediate Casing: 177.80 mm @ 801.00 m with 12.9 t Finished Drilling: 28-Jan-2022 Total Depth: 1818.00 m Rig Released: 28-Jan-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 28-Jan-2022 |
| Lic No.: 11667 | Corex Virden HZNTL A2-31-10-25 (WPM) UWI: 102.02-31-010-25W1.00 - Leg #1 UWI: 102.02-31-010-25W1.02 - Leg #2 – Added Please note added UWI for ReEntry UWI: 102.02-31-010-25W1.02 - Leg #2 Drilling Contractor: Ensign Drilling Inc. – Rig #14 UWI: 100.02-31-010-25W1.02 – Leg #2 Kick-Off Date: 29-Jan-2022 Kick-Off Point: 840.00 m Total Depth: 1520.00 m Rig Released: 30-Jan-2022 Status: Waiting On Service Rig (WOSR) Waiting on Service Rig: 30-Jan-2022 |
| Lic. No.: 11678 | Virden Roselea Unit No. 1 HZNTL 11-30-10-25 (WPM) UWI: 102.11-30-010-25W1.00 Spud Date: 31-Jan-2022 K.B. Elevation: 431.62 m Surface Casing: 244.50 mm @ 200.00 m with 15.0 Status: Drilling Ahead (DR) Drilling Ahead: 31-Jan-2022 |
| Lic. No.: 11680 | Daly Unit No. 4 HZNTL A7-35-9-28 (WPM) UWI: 103.07-35-009-28W1.00 Licence Issued: 25-Jan-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 9 Surface Location: 5C-35-9-28 Co-ords: 631.37 m N of S of Sec 35 50.00 m E of W of Sec 35 Grd Elev: 502.53 Proj. TD: 1670.00 m (Mississippian) Field: Sinclair Classification: Non Confidential Development Status: Location(LOC) Spud Date: 29-Jan-2022 K.B. Elevation: 502.53 m Surface Casing: 244.50 mm @ 140.00 m with 7.3 t Status: Drilling Ahead (DR) Drilling Ahead: 31-Jan-2022 |
| Lic. No.: 11681 | Daly Unit No. 4 HZNTL B7-35-9-28 (WPM) UWI: 104.07-35-009-28W1.00 Licence Issued: 25-Jan-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 9 Surface Location: 8A-34-9-28 Co-ords: 410.69 m N of S of Sec 34 60.00 m W of E of Sec 34 Grd Elev: 499.07 m Proj. TD: 1771.90 m (Mississippian) Field: Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11682 | Daly Unit No. 4 HZNTL A2-35-9-28 (WPM) UWI: 103.02-35-009-28W1.00 Licence Issued: 25-Jan-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 9 Surface Location: 1D-34-9-28 Co-ords: 390.69 m N of S of Sec 34 60.00 m W of E of Sec 34 Grd Elev: 498.88 m Proj. TD: 1794.30 m (Mississippian) Field: Sinclair Classification: Non Confidential Development Status: Location(LOC) |