| Natural Resources & Northern Development Petroleum Branch 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 T 204-945-6577 F 204-945-0586 www.manitoba.ca |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |
| (January 1/22 – February 28/22) |  |  |  |  |  |  |  |
| DRILLING ACTIVITY | To Feb 28/22 |  | To March 1/21 |  |  |  | 2021 Total |
| Drilling Licences Issued | 26 |  | 11 |  |  |  | 178 |
| Licences Cancelled | 0 |  | 0 |  |  |  | 9 |
| Vertical Wells Drilled | 1 |  | 0 |  |  |  | 9 |
| Horizontal Wells Drilled | 51 |  | 31 |  |  |  | 164 |
| Stratigraphic Test Holes Drilled | 0 |  | 0 |  |  |  | 1 |
| Wells Drilled - Total | 52 |  | 31 |  |  |  | 174 |
| No. of Metres Drilled | 112 198 |  | 70 370 |  |  |  | 372 739 |
| Wells Re-entered | 2 |  | 0 |  |  |  | 6 |
| Wells Being Drilled | 3 |  | 0 |  |  |  | 0 |
| No. of Active Rigs | 4 |  | 0 |  |  |  | 0 |
| Wells Licenced but Not Spudded | -25 |  | 35 |  |  |  | 62 |
| Wells Completed as Potential Oil Wells | 51 |  | 31 |  |  |  | 168 |
| Wells Abandoned Dry | 0 |  | 0 |  |  |  | 3 |
| Wells Drilled but Not Completed | 0 |  | 0 |  |  |  | 0 |
| Other Completions | 1 |  | 0 |  |  |  | 3 |
| New Wells on Production | 0 |  | 15 |  |  |  | 34 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |
| Geophysical Programs Licenced | 0 |  | 3 |  |  |  | 6 |
| Licences Cancelled | 0 |  | 0 |  |  |  | 0 |
| Kilometers Licenced | 0 |  | 59 |  |  |  | 439 |
| Kilometers Run | 0 |  | 55 |  |  |  | 438 |
|  | 2021 |  | 2020 |  |  |  |  |
| OIL PRICES (Average) | $/m3 ($/bbl) |  | $/m3 ($/bbl) |  |  |  |  |
| Month of November | 43 615.98 (97.88) |  | 303.27 (48.19) |  |  |  |  |
| Month of December | 43 615.98 (97.88) |  | 346.19 (55.01) |  |  |  |  |
| OIL PRODUCTION (M3) | 2022 |  | 2021 |  |  |  |  |
| Month of January | 183 146.2 |  | 175 786.3 |  |  |  |  |

WEEKLY WELL ACTIVITY REPORT

28 February 2022

| Lic. No.: 11497 | Corex Virden Prov. HZNTL 6-6-10-25 (WPM) UWI: 100.06-06-010-25W1.00 Status: Completing (COMP) Completing: 28-Feb-2022 |
| --- | --- |

| Lic. No.: 11649 | Tundra Daly Sinclair HZNTL D16-19-8-28 (WPM) UWI: 105.16-19-008-28W1.00 Status: Completing (COMP) Completing: 22-Feb-2022 |
| --- | --- |
| Lic. No.: 11663 | Tundra Daly Sinclair HZNTL A9-25-9-28 (WPM) UWI: 102.09-25-009-28W1.00 Status: Completing (COMP) Completing: 24-Feb-2022 |
| Lic. No.: 11670 | Tundra Daly Sinclair HZNTL 16-9-8-28 (WPM) UWI: 100.16-09-008-28W1.00 Spud Date: 25-Feb-2022 K.B. Elevation: 503.16 m Surface Casing: 244.50 mm @ 135.00 m with 8.0 t Status: Drilling Ahead (DR) Drilling Ahead: 28-Feb-2022 |
| Lic. No.: 11674 | Tundra Waskada Prov. HZNTL 15-22-1-26 (WPM) UWI: 100.15-22-001-26W1.00 Finished Drilling: 27-Feb-2022 Total Depth: 3131.00 m Production Casing: 139.70 mm @ 3131.00 m with 44.0 t Rig Released: 28-Feb-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 28-Feb-2022 |
| Lic. No.: 11675 | Tundra Waskada HZNTL 11-14-2-25 (WPM) UWI: 102.11-14-002-25W1.00 Status: Completing (COMP) Completing: 21-Feb-2022 |
| Lic. No.: 11677 | Tundra Waskada Prov. HZNTL A8-11-1-24 (WPM) UWI: 102.08-11-001-24W1.00 Status: Moving In Rotary Tools (MIRT) Moving In Rotary Tools: 28-Feb-2022 |
| Lic No.: 11678 | Virden Roselea Unit No. 1 HZNTL 11-30-10-25 (WPM) UWI: 102.11-30-010-25W1.00 Status: Completing (COMP) Completing: 24-Feb-2022 |
| Lic. No.: 11689 | Tundra Daly Sinclair HZNTL 13-31-9-29 (WPM) UWI: 100.13-31-009-29W1.00 Finished Drilling: 23-Feb-2022 Total Depth: 2180.00 m Production Casing: 139.70 mm @ 2180.00 m with 36.0 t Rig Released: 24-Feb-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 24-Feb-2022 |
| Lic. No.: 11692 | Tundra Daly Sinclair HZNTL A12-18-8-28 (WPM) UWI: 102.12-18-008-28W1.00 Finished Drilling: 24-Feb-2022 Total Depth: 2335.00 m Production Casing: 139.70 mm @ 2335.00 m with 36.5 t Rig Released: 24-Feb-2022 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 24-Feb-2022 |
| Lic. No.: 11694 | Corex Daly Sinclair Prov. HZNTL A3-11-9-28 (WPM) UWI: 102.03-11-009-28W1.00 Spud Date: 21-Feb-2022 K.B. Elevation: 493.21 m Surface Casing: 244.50 mm @ 149.00 m with 11.0 t Finished Drilling: 23-Feb-2022 Total Depth: 2381.00 m Production Casing: 139.70 mm @ 2381.00m with 44 t Rig Released: 25-Feb-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 25-Feb-2022 |
| Lic. No.: 11695 | Corex Daly Sinclair Prov. HZNTL A4-11-9-28 (WPM) UWI: 102.04-11-009-28W1.00 Spud Date: 25-Feb-2022 K.B. Elevation: 493.22 m Surface Casing: 244.50 mm @ 147.00 m with 11.0 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 28-Feb-2022 |
| Lic. No.: 11698 | Tundra Daly Sinclair SWD A12-24-10-29 (WPM) UWI: 102.12-24-010-29W1.00 Licence Issued: 22-Feb-2022 Licensee: Tundra Oil and Gas Limited Mineral Rights: Tundra Oil and Gas Limited Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 12D-24-10-29 Co-ords: 462.71 m S of N of Sec 12 280.00 m E of W of Sec 12 Grd Elev: 522.76 m Proj. TD: 1751.40 m (Silurian) Field: Daly Sinclair Classification: Deeper Pool Wildcat Status: Location(LOC) Spud Date: 24-Feb-2022 K.B. Elevation: 526.96 m Surface Casing: 244.50 mm @ 225.00 m with 13.0 t Intermediate Casing: 177.80 mm @ 1390.00 m with 21.0 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 28-Feb-2022 |
| Lic. No.: 11699 | Tundra Waskada HZNTL 3-8-1-25 (WPM) UWI: 100.03-08-001-25W1.00 Licence Issued: 24-Feb-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 10 Surface Location: 5A-7-1-25 Co-ords: 495.27 m N of S of Sec 7 273.76 m E of W of Sec 7 Grd Elev: 464.05 m Proj. TD: 2939.30 m (Triassic) Field: Waskada Classification: Non Confidential Development Status: Location(LOC) |