| Natural Resources & Northern Development Petroleum Branch 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 T 204-945-6577 F 204-945-0586 www.manitoba.ca |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |
| (January 1/22 – Aug 8/22) |  |  |  |  |  |  |  |
| DRILLING ACTIVITY | To Aug 8/22 |  | To Aug 9/21 |  |  |  | 2021 Total |
| Drilling Licences Issued | 124 |  | 101 |  |  |  | 178 |
| Licences Cancelled | 0 |  | 0 |  |  |  | 9 |
| Vertical Wells Drilled | 2 |  | 6 |  |  |  | 9 |
| Horizontal Wells Drilled | 111 |  | 70 |  |  |  | 164 |
| Stratigraphic Test Holes Drilled | 0 |  | 1 |  |  |  | 1 |
| Wells Drilled - Total | 113 |  | 77 |  |  |  | 174 |
| No. of Metres Drilled | 253 100 |  | 157 197 |  |  |  | 372 739 |
| Wells Re-entered | 3 |  | 1 |  |  |  | 6 |
| Wells Being Drilled | 4 |  | 4 |  |  |  | 0 |
| No. of Active Rigs | 4 |  | 4 |  |  |  | 0 |
| Wells Licenced but Not Spudded | 6 |  | 74 |  |  |  | 62 |
| Wells Completed as Potential Oil Wells | 111 |  | 73 |  |  |  | 168 |
| Wells Abandoned Dry | 0 |  | 3 |  |  |  | 3 |
| Wells Drilled but Not Completed | 0 |  | 0 |  |  |  | 0 |
| Other Completions | 2 |  | 1 |  |  |  | 3 |
| New Wells on Production | 0 |  | 43 |  |  |  | 34 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |
| Geophysical Programs Licenced | 0 |  | 3 |  |  |  | 6 |
| Licences Cancelled | 0 |  | 0 |  |  |  | 0 |
| Kilometers Licenced | 0 |  | 59 |  |  |  | 439 |
| Kilometers Run | 0 |  | 55 |  |  |  | 438 |
|  | 2022 |  | 2021 |  |  |  |  |
| OIL PRICES (Average) | $/m3 ($/bbl) |  | $/m3 ($/bbl) |  |  |  |  |
| Month of June | 43 924.29 (146.87) |  | 43 529.96 (84.21) |  |  |  |  |
| Month of July | 43 776.40 (123.37) |  | 43 534.80 (84.98) |  |  |  |  |
| OIL PRODUCTION (M3) | 2022 |  | 2021 |  |  |  |  |
| Month of July | 177 685.0 |  | 171 983.7 |  |  |  |  |

WEEKLY WELL ACTIVITY REPORT

08 August 2022

| Lic. No.: 11635 | Melita Pierson HZNTL 5-21-1-27 (WPM) UWI: 102.05-21-001-27W1.00 Status: Completing (COMP) Completing: 02-Aug-2022 |
| --- | --- |
| Lic. No.: 11722 | Daly Unit No. 15 HZNTL 9-9-10-28 (WPM) UWI: 102.09-09-010-28W1.00 Spud Date: 02-Aug-2022 K.B. Elevation: 519.48 m Surface Casing: 244.50 mm @ 149.00 m with 9.0 t Finished Drilling: 06-Aug-2022 Total Depth: 2585.00 m Production Casing: 139.70 mm @ 2585.00 m with 45.0 t Rig Released: 07-Aug-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 07-Aug-2022 |
| Lic. No.: 11733 | Tundra Daly Sinclair HZNTL 13-31-8-29 (WPM) UWI: 100.13-31-008-29W1.00 Status: Completing (COMP) Completing: 03-Aug-2022 |
| Lic. No.: 11741 | Tundra Waskada HZNTL 1-22-1-24 (WPM) UWI: 100.01-22-001-24W1.00 Finished Drilling: 01-Aug-2022 Total Depth: 2397.00 m Production Casing: 139.70 mm @ 2397.00 m with 30.5 t Rig Released: 02-Aug-2022 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 02-Aug-2022 |
| Lic. No.: 11742 | Sinclair Unit No. 2 HZNTL 1-19-7-28 (WPM) UWI: 102.01-19-007-28W1.00 Intermediate Casing: 177.80 mm @ 1081.00 m with 22.0 t Finished Drilling: 05-Aug-2022 Total Depth: 2477.00 m Rig Released: 05-Aug-2022 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 05-Aug-2022 |
| Lic. No.: 11752 | Corex Daly Sinclair HZNTL B1-8-9-28 (WPM) UWI: 103.01-08-009-28W1.00 Spud Date: 07-Aug-2022 K.B. Elevation: 506.31 m Surface Casing: 244.50 mm @ 149.00 m with 9.0 t Status: Drilling Ahead (DR) Drilling Ahead: 08-Aug-2022 |
| Lic. No.: 11760 | Melita Pierson HZNTL 15-25-1-28 (WPM) UWI: 100.15-25-001-28W1.00 Finished Drilling: 04-Aug-2022 Total Depth: 1670.00 m Liner: 139.70 mm @ 1670.00 m with 40.0 t Rig Released: 05-Aug-2022 Status: Completing (COMP) Completing: 08-Aug-2022 |
| Lic. No.: 11763 | Tundra Daly Sinclair HZNTL 1-33-10-28 (WPM) UWI: 100.01-33-010-28W1.00 Production Casing: 139.70 mm @ 2275.00 m with 23.5 t Rig Released: 02-Aug-2022 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 02-Aug-2022 |
| Lic. No.: 11766 | Corex Daly Sinclair HZNTL A1-7-10-27 (WPM) UWI: 102.01-07-010-27W1.00 Status: Completing (COMP) Completing: 04-Aug-2022 |
| Lic. No.: 11767 | Corex Daly Sinclair HZNTL A2-7-10-27 (WPM) UWI: 102.02-07-010-27W1.00 Production Casing: 139.70 m @ 2345.00 m with 40.0 t Rig Released: 02-Aug-2022 Status: Completing (COMP) Completing: 06-Aug-2022 |
| Lic. No.: 11771 | Daly Unit No. 11 HZNTL A13-27-10-29 (WPM) UWI: 103.13-27-010-29W1.00 Spud Date: 07-Aug-2022 K.B. Elevation: 527.12 m Surface Casing: 244.50 mm @ 136.00 m with 7.5 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 08-Aug-2022 |
| Lic. No.: 11774 | Tundra Daly Sinclair HZNTL 1-28-10-28 (WPM) UWI: 100.01-28-010-28W1.00 Spud Date: 02-Aug-2022 K.B. Elevation: 517.01 m Surface Casing: 244.50 mm @ 149.00 m with 7.5 t Finished Drilling: 06-Aug-2022 Total Depth: 2104.00 m Production Casing: 139.70 mm @ 2104.00 m with 31.5 t Rig Released: 07-Aug-2022 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 07-Aug-2022 |
| Lic. No.: 11775 | Tundra et al Daly Sinclair HZNTL A12-21-9-29 (WPM) UWI: 104.12-21-009-29W1.00 Status: Completing (COMP) Completing: 02-Aug-2022 |
| Lic. No.: 11776 | Tundra et al Daly Sinclair HZNTL 13-21-9-29 (WPM) UWI: 102.13-21-009-29W1.00 Status: Completing (COMP) Completing: 05-Aug-2022 |
| Lic. No.: 11777 | Tundra et al Daly Sinclair HZNTL A13-21-9-29 (WPM) UWI: 103.13-21-009-29W1.00 Status: Completing (COMP) Completing: 03-Aug-2022 |
| Lic. No.: 11786 | Tundra Waskada HZNTL 13-5-1-25 (WPM) UWI: 100.13-05-001-25W1.02 Status: Completing (COMP) Completing: 02-Aug-2022 |
| Lic. No.: 11788 | Tundra Waskada Prov. HZNTL A15-16-1-24 (WPM) UWI: 102.15-16-001-24W1.00 Please note revision to Report dated 18-Jul-2022 Drilling Contractor: Ensign Drilling Inc. – Rig 10 Spud Date: 03-Aug-2022 K.B. Elevation: 511.42 m Surface Casing: 219.10 mm @ 139.00 m with 5.5 t Finished Drilling: 07-Aug-2022 Total Depth: 2923.00 m Production Casing: 139.70 mm @ 2923.00 m with 36.6 t Rig Released: 08-Aug-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 08-Aug-2022 |
| Lic. No.: 11791 | Daly Unit No. 3 HZNTL C15-12-10-28 (WPM) UWI: 105.15-12-010-28W1.00 Licence Issued: 02-Aug-2022 Licensee: Corex Resources Ltd. Mineral Rights: Corex Resources Ltd. Contractor: Ensign Drilling Inc. - Rig# 14 Surface Location: 15D-11-10-28 Co-ords: 173.03 m S of N of Sec 11 603.59 m W of E of Sec 11 Grd Elev: 497.57 m Proj. TD: 2120.07 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11792 | Ewart Unit No. 10 HZNTL A2-5-8-28 (WPM) UWI: 102.02-05-008-28W1.00 Licence Issued: 02-Aug-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 6A-4-8-28 Co-ords: 527.74 m N of S of Sec 4 713.10 m E of W of Sec 4 Grd Elev: 496.02 m Proj. TD: 2306.70 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11793 | Ewart Unit No. 2 Prov. HZNTL A1-29-7-28 (WPM) UWI: 102.01-29-007-28W1.00 Licence Issued: 02-Aug-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 1A-30-7-28 Co-ords: 60.00 m N of S of Sec 30 80.00 m W of E of Sec 30 Grd Elev: 492.11 m Proj. TD: 2507.20 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) Spud Date: 05-Aug-2022 K.B. Elevation: 495.95 m Surface Casing: 244.50 mm @ 136.00 m with 8.0 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 08-Aug-2022 |
| Lic. No.: 11794 | Tundra Waskada HZNTL C3-5-1-24 (WPM) UWI: 105.03-05-001-24W1.00 Licence Issued: 02-Aug-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 10 Surface Location: 10A-5-1-24 Co-ords: 710.00 m S of N of Sec 5 540.00 m W of E of Sec 5 Grd Elev: 488.52 m Proj. TD: 1765.50 m (Mississippian) Field: Waskada Classification: Non Confidential Development Status: Location(LOC) UWI: 105.03-05-001-24W1.00 Spud Date: 08-Aug-2022 K.B. Elevation: 492.32 m Surface Casing: 244.50 mm @ 136.00 m with 6.5 t Status: Drilling Ahead (DR) Drilling Ahead: 08-Aug-2022 |
| Lic. No.: 11795 | Tundra Daly Sinclair HZNTL B4-28-9-29 (WPM) UWI: 103.04-28-009-29W1.00 Licence Issued: 03-Aug-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 1D-28-9-29 Co-ords: 299.03 m N of S of Sec 28 60.00 m W of E of Sec 28 Grd Elev: 531.99 m Proj. TD: 2185.40 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11796 | Tundra Daly Sinclair HZNTL D4-30-10-28 (WPM) UWI: 105.04-30-010-28W1.00 Licence Issued: 03-Aug-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 1A-30-10-28 Co-ords: 66.08 m N of S of Sec 30 60.00 m W of E of Sec 30 Grd Elev: 515.41 m Proj. TD: 2084.80 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11797 | Tundra Daly Sinclair HZNTL C13-19-10-28 (WPM) UWI: 104.13-19-010-28W1.00 Licence Issued: 05-Aug-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 13C-20-10-28 Co-ords: 161.74 m S of N of Sec 20 60.00 m E of W of Sec 20 Grd Elev: 514.98 m Proj. TD: 2262.40 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11798 | Tundra Daly Sinclair HZNTL 9-34-9-29 (WPM) UWI: 102.09-34-009-29W1.00 Licence Issued: 05-Aug-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 9D-33-9-29 Co-ords: 457.14 m S of N of Sec 33 60.00 m W of E of Sec 33 Grd Elev: 526.88 m Proj. TD: 2324.90 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11799 | Melita Pierson HZNTL 16-36-1-29 (WPM) UWI: 103.16-36-001-29W1.00 Licence Issued: 05-Aug-2022 Licensee: Melita Resources Ltd. Mineral Rights: Melita Resources Ltd. Contractor: Betts Drilling Ltd. - Rig# 1 Surface Location: 14A-36-1-29 Co-ords: 260.47 m S of N of Sec 36 667.50 m E of W of Sec 36 Grd Elev: 463.66 m Proj. TD: 1768.19 m (Triassic) Field: Pierson Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11800 | Tundra Daly Sinclair Prov. HZNTL 9-6-10-27 (WPM) UWI: 100.09-06-010-27W1.00 Licence Issued: 05-Aug-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 8D-1-10-28 Co-ords: 745.01 m N of S of Sec 1 60.00 m W of E of Sec 1 Grd Elev: 492.02 m Proj. TD: 2249.90 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |