| Natural Resources & Northern Development Petroleum Branch 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 T 204-945-6577 F 204-945-0586 www.manitoba.ca |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |
| (January 1/22 – Oct 10/22) |  |  |  |  |  |  |  |
| DRILLING ACTIVITY | To Oct 10/22 |  | To Oct 11/21 |  |  |  | 2021 Total |
| Drilling Licences Issued | 165 |  | 122 |  |  |  | 178 |
| Licences Cancelled | 2 |  | 6 |  |  |  | 9 |
| Vertical Wells Drilled | 4 |  | 9 |  |  |  | 9 |
| Horizontal Wells Drilled | 163 |  | 123 |  |  |  | 164 |
| Stratigraphic Test Holes Drilled | 0 |  | 1 |  |  |  | 1 |
| Wells Drilled - Total | 167 |  | 133 |  |  |  | 174 |
| No. of Metres Drilled | 376 057 |  | 276 081 |  |  |  | 372 739 |
| Wells Re-entered | 3 |  | 3 |  |  |  | 6 |
| Wells Being Drilled | 5 |  | 2 |  |  |  | 0 |
| No. of Active Rigs | 5 |  | 3 |  |  |  | 0 |
| Wells Licenced but Not Spudded | -5 |  | 45 |  |  |  | 62 |
| Wells Completed as Potential Oil Wells | 164 |  | 127 |  |  |  | 168 |
| Wells Abandoned Dry | 0 |  | 3 |  |  |  | 3 |
| Wells Drilled but Not Completed | 0 |  | 0 |  |  |  | 0 |
| Other Completions | 3 |  | 3 |  |  |  | 3 |
| New Wells on Production | 0 |  | 100 |  |  |  | 34 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |
| Geophysical Programs Licenced | 0 |  | 6 |  |  |  | 6 |
| Licences Cancelled | 0 |  | 0 |  |  |  | 0 |
| Kilometers Licenced | 0 |  | 439 |  |  |  | 439 |
| Kilometers Run | 0 |  | 55 |  |  |  | 438 |
|  | 2022 |  | 2021 |  |  |  |  |
| OIL PRICES (Average) | $/m3 ($/bbl) |  | $/m3 ($/bbl) |  |  |  |  |
| Month of Aug | 43 725.30 (115.25) |  | 43 510.89 (81.18) |  |  |  |  |
| Month of Sep | 43 693.58 (110.21) |  | 43 533.73 (84.81) |  |  |  |  |
| OIL PRODUCTION (M3) | 2022 |  | 2021 |  |  |  |  |
| Month of Sep | 182 816.0 |  | 180 260.4 |  |  |  |  |

WEEKLY WELL ACTIVITY REPORT

10 October 2022

| Lic. No.: 11669 | Sinclair Unit No. 2 HZNTL B12-25-7-29 (WPM) UWI: 104.12-25-007-29W1.00 - Status: Completing (COMP) Completing: 08-Oct-2022 |
| --- | --- |
| Lic. No.: 11673 | Tundra Manson HZNTL 1-23-13-28 (WPM) UWI: 100.01-23-013-28W1.00 Status: Completing (COMP) Completing: 04-Oct-2022 |
| Lic. No.: 11683 | Corex Virden HZNTL 10-12-10-26 (WPM) UWI: 100.10-12-010-26W1.00 - Leg #1 UWI: 100.10-12-010-26W1.02 - Leg #2 Spud Date: 09-Oct-2022 K.B. Elevation: 439.95 m Surface Casing: 244.50 mm @ 201.00 m with 12.0 t Status: Drilling Ahead (DR) Drilling Ahead: 10-Oct-2022 |
| Lic. No.: 11740 | Ewart Unit No. 18 Prov. HZNTL C1-6-9-28 (WPM) UWI: 104.01-06-009-28W1.00 Spud Date: 03-Oct-2022 K.B. Elevation: 515.00 m Surface Casing: 244.50 mm @ 135.00 m with 6.5 t Finished Drilling: 06-Oct-2022 Total Depth: 2547.00 m Production Casing: 139.70 mm @ 2547.00 m with 39.1 t Rig Released: 07-Oct-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 07-Oct-2022 |
| Lic. No.: 11748 | Routledge Unit No. 1 HZNTL A6-33-9-25 (WPM) UWI: 103.06-33-009-25W1.00 Spud Date: 05-Oct-2022 K.B. Elevation: 438.33 m Surface Casing: 244.50 mm @ 149.00 m with 9.0 t Intermediate Casing: 177.80 mm @ 785.00 m with 13.5 t Finished Drilling: 08-Oct-2022 Total Depth: 1294.00 m Rig Released: 08-Oct-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 08-Oct-2022 |
| Lic. No.: 11749 | Corex Virden HZNTL 11-23-9-25 (WPM) UWI: 102.11-23-009-25W1.00 Finished Drilling: 04-Oct-2022 Total Depth: 1661.00 m Rig Released: 05-Oct-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 05-Oct-2022 |
| Lic. No.: 11758 | Corex Virden HZNTL A3-13-11-26 (WPM) UWI: 103.03-13-011-26W1.00 Status: Completing (COMP) Completing: 06-Oct-2022 |
| Lic. No.: 11779 | Sinclair Unit No. 17 HZNTL B8-19-8-28 (WPM) UWI: 104.08-19-008-28W1.00 Status: Completing (COMP) Completing: 09-Oct-2022 |
| Lic. No.: 11781 | Ewart Unit No. 18 HZNTL D1-1-9-29 (WPM) UWI: 105.01-01-009-29W1.00 Spud Date: 07-Oct-2022 K.B. Elevation: 525.14 m Surface Casing: 244.50 mm @ 135.00 m with 6.5 t Intermediate Casing: 177.80 mm @ 1037.00 m with 20.5 t Status: Drilling Ahead (DR) Drilling Ahead: 10-Oct-2022 |
| Lic. No.: 11804 | Tundra Waskada HZNTL A8-14-2-25 (WPM) UWI: 103.08-14-002-25W1.00 Spud Date: 04-Oct-2022 K.B. Elevation: 476.81 m Surface Casing: 219.10 mm @ 147.00 m with 5.5 t Finished Drilling: 07-Oct-2022 Total Depth: 2062.00 m Production Casing: 139.70 mm @ 2062.00 m with 26.0 t Rig Released: 08-Oct-2022 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 08-Oct-2022 |
| Lic. No.: 11813 | Sinclair Unit No. 19 HZNTL 9-23-8-29 (WPM) UWI: 102.09-23-008-29W1.00 Status: Completing (COMP) Completing: 07-Oct-2022 |
| Lic. No.: 11815 | Tundra Pierson HZNTL 10-30-1-28 (WPM) UWI: 100.10-30-001-28W1.00 Status: Completing (COMP) Completing: 04-Oct-2022 |
| Lic. No.: 11821 | Surge Daly Sinclair HZNTL B12-15-8-28 (WPM) UWI: 103.12-15-008-28W1.00 Please note revision to Report dated 29-Aug-2022 Drilling Contractor: Ensign Drilling Inc. Rig #360 Spud Date: 05-Oct-2022 K.B. Elevation: 491.24 m Surface Casing: 244.50 mm @ 143.00 m with 13.0 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 10-Oct-2022 |
| Lic. No.: 11825 | Tundra et al Waskada HZNTL A1-14-2-25 (WPM) UWI: 102.01-14-002-25W1.00 Finished Drilling: 03-Oct-2022 Total Depth: 1996.00 m Production Casing: 139.70 mm @ 1996.00 m with 25.0 t Rig Released: 03-Oct-2022 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 04-Oct-2022 |
| Lic. No.: 11828 | Waskada Unit No. 16 Prov. HZNTL 14-10-2-25 (WPM) UWI: 102.14-10-002-25W1.00 Spud Date: 08-Oct-2022 K.B. Elevation: 476.69 m Surface Casing: 219.10 mm @ 144.00 m with 5.5 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 10-Oct-2022 |
| Lic. No.: 11832 | Tundra Virden HZNTL A4-20-11-26 (WPM) UWI: 103.04-20-011-26W1.00 Finished Drilling: 04-Oct-2022 Total Depth: 2147.00 m Production Casing: 139.70 mm @ 2147.00 m with 29.5 t Rig Released: 05-Oct-2022 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 05-Oct-2022 |
| Lic. No.: 11833 | Tundra Virden HZNTL B4-20-11-26 (WPM) UWI: 104.01-19-011-26W1.02 Status: Completing (COMP) Completing: 06-Oct-2022 |
| Lic. No.: 11834 | Tundra Daly Sinclair Prov. HZNTL B12-4-10-29 (WPM) UWI: 103.12-04-010-29W1.00 Spud Date: 05-Oct-2022 K.B. Elevation: 524.21 m Finished Drilling: 08-Oct-2022 Total Depth: 2321.00 m Production Casing: 139.70 mm @ 2321.0 m with 32.5 t Rig Released: 09-Oct-2022 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 09-Oct-2022 |
| Lic. No.: 11838 | Tundra et al Daly Sinclair HZNTL A1-1-10-29 (WPM) UWI: 102.01-01-010-29W1.00 Spud Date: 09-Oct-2022 K.B. Elevation: 526.53 m Surface Casing: 244.50 mm @ 136.00 m with 7.5 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 10-Oct-2022 |
| Lic. No.: 11841 | Tundra Daly Sinclair HZNTL C9-4-9-28 (WPM) UWI: 104.09-04-009-28W1.00 Licence Issued: 06-Oct-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 9A-5-9-28 Co-ords: 685.11 m S of N of Sec 5 69.14 m W of E of Sec 5 Grd Elev: 486.14 m Proj. TD: 2324.40 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11842 | Corex Daly Sinclair HZNTL A8-8-9-28 (WPM) UWI: 102.08-08-009-28W1.00 Licence Issued: 06-Oct-2022 Licensee: Corex Resources Ltd Mineral Rights: Corex Resources Ltd Contractor: Ensign Drilling Inc. - Rig# 14 Surface Location: 8C-7-9-28 Co-ords: 640.00 m N of S of Sec 7 205.00 m W of E of Sec 7 Grd Elev: 502.87 m Proj. TD: 2511.44 m (Devonian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11843 | Daly Unit No. 11 HZNTL A5-27-10-29 (WPM) UWI: 102.05-27-010-29W1.00 Licence Issued: 07-Oct-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 12B-26-10-29 Co-ords: 624.42 m S of N of Sec 26 60.00 m E of W of Sec 26 Grd Elev: 522.98 m Proj. TD: 2388.50 m (Devonian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11844 | Tundra Daly Sinclair HZNTL 1-34-9-29 (WPM) UWI: 100.01-34-009-29W1.00 Licence Issued: 07-Oct-2022 Licensee: Tundra Oil and Gas Limited Mineral Rights: Tundra Oil and Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 1D-33-9-29 Co-ords: 302.79 m N of S of Sec 33 60.00 m W of E of Sec 33 Grd Elev: 532.27 m Proj. TD: 2330.00 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |