| Natural Resources & Northern Development Petroleum Branch 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 T 204-945-6577 F 204-945-0586 www.manitoba.ca |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |
| (January 1/22 – Oct 24/22) |  |  |  |  |  |  |  |
| DRILLING ACTIVITY | To Oct 24/22 |  | To Oct 25/21 |  |  |  | 2021 Total |
| Drilling Licences Issued | 174 |  | 130 |  |  |  | 178 |
| Licences Cancelled | 2 |  | 6 |  |  |  | 9 |
| Vertical Wells Drilled | 4 |  | 9 |  |  |  | 9 |
| Horizontal Wells Drilled | 177 |  | 129 |  |  |  | 164 |
| Stratigraphic Test Holes Drilled | 0 |  | 1 |  |  |  | 1 |
| Wells Drilled - Total | 181 |  | 139 |  |  |  | 174 |
| No. of Metres Drilled | 409 057 |  | 290 857 |  |  |  | 372 739 |
| Wells Re-entered | 6 |  | 3 |  |  |  | 6 |
| Wells Being Drilled | 3 |  | 3 |  |  |  | 0 |
| No. of Active Rigs | 4 |  | 3 |  |  |  | 0 |
| Wells Licenced but Not Spudded | -8 |  | 46 |  |  |  | 62 |
| Wells Completed as Potential Oil Wells | 178 |  | 133 |  |  |  | 168 |
| Wells Abandoned Dry | 0 |  | 3 |  |  |  | 3 |
| Wells Drilled but Not Completed | 0 |  | 0 |  |  |  | 0 |
| Other Completions | 3 |  | 3 |  |  |  | 3 |
| New Wells on Production | 0 |  | 100 |  |  |  | 34 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |
| Geophysical Programs Licenced | 0 |  | 6 |  |  |  | 6 |
| Licences Cancelled | 0 |  | 0 |  |  |  | 0 |
| Kilometers Licenced | 0 |  | 439 |  |  |  | 439 |
| Kilometers Run | 0 |  | 55 |  |  |  | 438 |
|  | 2022 |  | 2021 |  |  |  |  |
| OIL PRICES (Average) | $/m3 ($/bbl) |  | $/m3 ($/bbl) |  |  |  |  |
| Month of Aug | 43 725.30 (115.25) |  | 43 510.89 (81.18) |  |  |  |  |
| Month of Sep | 43 693.58 (110.21) |  | 43 533.73 (84.81) |  |  |  |  |
| OIL PRODUCTION (M3) | 2022 |  | 2021 |  |  |  |  |
| Month of Sep | 182 816.0 |  | 180 260.4 |  |  |  |  |

WEEKLY WELL ACTIVITY REPORT

24 October 2022

| Lic. No.: 11764 | Corex Virden HZNTL 5-6-10-25 (WPM) UWI: 102.05-06-010-25W1.00 Spud Date: 20-Oct-2022 K.B. Elevation: 440.06 m Surface Casing: 244.50 mm @ 148.00 m with 9.0 t Intermediate Casing: 177.80 mm @ 805.00 m with 13.5 t Finished Drilling: 24-Oct-2022 Total Depth: 1985.00 m Rig Released: 24-Oct-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 24-Oct-2022 |
| --- | --- |
| Lic. No.: 11765 | Corex Virden HZNTL A5-6-10-25 (WPM) UWI: 103.05-06-010-25W1.00 Spud Date: 24-Oct-2022 K.B. Elevation: 439.90 m Status: Drilling Ahead (DR) Drilling Ahead: 24-Oct-2022 |
| Lic. No.: 11768 | Corex Virden HZNTL 8-12-10-26 (WPM) UWI: 100.08-12-010-26W1.00 Intermediate Casing: 177.80 mm @ 759.00 m with 12.5 t Finished Drilling: 19-Oct-2022 Total Depth: 1365.00 m Rig Released: 20-Oct-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 20-Oct-2022 |
| Lic. No.: 11800 | Tundra Daly Sinclair Prov. HZNTL 9-6-10-27 (WPM) UWI: 100.09-06-010-27W1.00 Please note revision to Report dated 08-Aug-2022 Drilling Contractor: Ensign Drilling Inc. Rig #12 Spud Date: 17-Oct-2022 K.B. Elevation: 495.56 m Surface Casing: 244.50 mm @ 136.00 m with 7.5 t Finished Drilling: 20-Oct-2022 Total Depth: 2248.00 m Production Casing: 139.70 mm @ 2248.00 m with 31.2 t Rig Released: 21-Oct-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 21-Oct-2022 |
| Lic. No.: 11822 | Surge Daly Sinclair HZNTL C12-15-8-28 (WPM) UWI: 104.12-15-008-28W1.00 Finished Drilling: 18-Oct-2022 Total Depth: 2407.00 m Production Casing: 144.30 mm @ 2407.0 m with 34.5 t Rig Released: 19-Oct-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 19-Oct-2022 |
| Lic. No.: 11823 | Surge Daly Sinclair HZNTL D12-15-8-28 (WPM) UWI: 105.12-15-008-28W1.00 Please note revision to Report dated 29-Aug-2022 Drilling Contractor: Ensign Drilling Inc. Rig #360 Spud Date: 19-Oct-2022 K.B. Elevation: 490.99 m Surface Casing: 244.50 mm @ 143.00 m with 12.0 t Finished Drilling: 23-Oct-2022 Total Depth: 2357.00 m Production Casing: 114.30 mm @ 2357.00 m with 41.5 t Rig Released: 24-Oct-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 24-Oct-2022 |
| Lic. No.: 11825 | Tundra et al Waskada HZNTL A1-14-2-25 (WPM) UWI: 102.01-14-002-25W1.00 Status: Completing (COMP) Completing: 18-Oct-2022 |
| Lic. No.: 11831 | Waskada et al Unit No. 17 HZNTL A3-15-2-25 (WPM) UWI: 104.03-15-002-25W1.00 Spud Date: 17-Oct-2022 K.B. Elevation: 474.89 m Surface Casing: 219.10 mm @ 142.00 m with 5.5 t Finished Drilling: 22-Oct-2022 Total Depth: 3250.00 m Production Casing: 139.70 mm @ 3250.00 m with 40.0 t Rig Released: 23-Oct-2022 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 23-Oct-2022 |
| Lic. No.: 11835 | Tundra Daly Sinclair HZNTL A9-34-9-29 (WPM) UWI: 103.09-34-009-29W1.00 Spud Date: 19-Oct-2022 K.B. Elevation: 536.97 m Surface Casing: 244.50 mm @ 135.00 m with 6.5 t Finished Drilling: 22-Oct-2022 Total Depth: 2359.00 m Production Casing: 139.70 mm @ 2359.00 m with 37.0 t Rig Released: 24-Oct-2022 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 24-Oct-2022 |
| Lic. No.: 11843 | Daly Unit No. 11 HZNTL A5-27-10-29 (WPM) UWI: 102.05-27-010-29W1.00 Spud Date: 21-Oct-2022 K.B. Elevation: 526.93 m Surface Casing: 244.50 mm @ 135.00 m with 6.5 t Status: Drilling Ahead (DR) Drilling Ahead: 24-Oct-2022 |
| Lic. No.: 11844 | Tundra Daly Sinclair HZNTL 1-34-9-29 (WPM) UWI: 100.01-34-009-29W1.00 Please note revision to Report dated 29-Aug-2022 Drilling Contractor: Ensign Drilling Inc. Rig # 360 Status: Moving in Rotary Tools (MIRT) Moving in Rotary Tools: 24-Oct-2022 |
| Lic. No.: 11847 | Waskada Unit No. 2 HZNTL B13-27-1-26 (WPM) UWI: 105.13-27-001-26W1.00 Spud Date: 23-Oct-2022 K.B. Elevation: 467.89 m Surface Casing: 219.10 mm @ 144.00 m with 5.5 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 24-Oct-2022 |
| Lic. No.: 11850 | Tundra Waskada Prov. HZNTL 2-18-2-25 (WPM) UWI: 102.02-18-002-25W1.00 Licence Issued: 20-Oct-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 10 Surface Location: 4C-16-2-25 Co-ords: 300.00 m N of S of Sec 4 60.00 m E of W of Sec 4 Grd Elev: 471.61 m Proj. TD: 3208.30 m (Triassic) Field: Waskada Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11851 | Tundra Daly Sinclair HZNTL C1-25-10-29 (WPM) UWI: 104.01-25-010-29W1.00 Licence Issued: 20-Oct-2022 Licensee: Tundra Oil and Gas Limited Mineral Rights: Tundra Oil and Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 1A-26-10-29 Co-ords: 240.00 m N of S of Sec 26 60.00 m W of E of Sec 26 Grd Elev: 521.97 m Proj. TD: 2305.60 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11852 | Daly Unit No. 19 HZNTL A16-25-10-29 (WPM) UWI: 102.16-25-010-29W1.00 Licence Issued: 20-Oct-2022 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 16A-25-10-29 Co-ords: 342.62 m S of N of Sec 26 60.00 m W of E of Sec 26 Grd Elev: 522.02 m Proj. TD: 2383.10 m (Devonian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11853 | Daly Unit No. 19 HZNTL A1-36-10-29 (WPM) UWI: 102.01-36-010-29W1.00 Licence Issued: 20-Oct-2022 Licensee: Tundra Oil and Gas Limited Mineral Rights: Tundra Oil and Gas Limited Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 16D-26-10-29 Co-ords: 69.57 m S of N of Sec 26 60.00 m W of E of Sec 26 Grd Elev: 522.32 m Proj. TD: 2403.50 m (Devonian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |