| Economic Development, Investment, Trade & Natural Resources |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Petroleum Branch |  |  |  |  |  |  |  |  |
| 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 |  |  |  |  |  |  |  |  |
| T 204-945-6577 F 204-945-0586 |  |  |  |  |  |  |  |  |
| www.manitoba.ca |  |  |  |  |  |  |  |  |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| (January 1/23 -- February 27/23) |  |  |  |  |  |  |  |  |
| DRILLING ACTIVITY |  |  | To Feb 27/23 |  |  | ToFeb 28/22 |  | 2022 Total |
| Drilling Licences Issued |  |  | 29 |  |  | 26 |  | 222 |
| Licences Cancelled |  |  | 2 |  |  | 0 |  | 11 |
| Vertical Wells Drilled |  |  | 0 |  |  | 1 |  | 7 |
| Horizontal Wells Drilled |  |  | 42 |  |  | 50 |  | 220 |
| Stratigraphic Test Holes Drilled |  |  | 1 |  |  | 0 |  | 1 |
| Wells Drilled - Total |  |  | 43 |  |  | 51 |  | 228 |
| No. of Metres Drilled |  | 93512 |  |  | 112198 |  |  | 503107 |
| Wells Re-entered |  |  | 0 |  |  | 2 |  | 7 |
| Wells Being Drilled |  |  | 4 |  |  | 3 |  | 0 |
| No. of Active Rigs |  |  | 0 |  |  | 4 |  | 0 |
| Wells Licenced but Not Spudded |  |  | 30 |  |  | 37 |  | 46 |
| Wells Completed as Potential Oil Wells |  |  | 42 |  |  | 50 |  | 221 |
| Wells Abandoned Dry |  |  | 1 |  |  | 0 |  | 1 |
| Wells Drilled but Not Completed |  |  | 0 |  |  | 0 |  | 0 |
| Other Completions |  |  | 0 |  |  | 1 |  | 3 |
| New Wells on Production |  |  | 0 |  |  | 0 |  | 0 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |  |
| Geophysical Programs Licenced |  |  | 0 |  |  | 3 |  | 0 |
| Licences Cancelled |  |  | 0 |  |  | 0 |  | 0 |
| Kilometers Licenced |  |  | 0 |  |  | 59.00 |  | 0 |
| Kilometers Run |  |  | 0 |  |  | 55 |  | 0 |
|  |  |  | 2022/2023 |  |  | 2021/2022 |  |  |
| OIL PRICES (Average) |  | $/M3 ¦ $/BBL |  |  | $/M3 ¦ $/BBL |  |  |  |
| Month of December |  | 596.10 | 94.72 |  | 535.62 | 85.11 |  |  |
| Month of January |  | 615.54 | 97.81 |  | 613.66 | 97.51 |  |  |
| OIL PRODUCTION (M3) |  |  | 2023 |  |  | 2022 |  |  |
| Month of January |  | 202,436.8 |  |  | 182,749.0 |  |  |  |

WEEKLY WELL ACTIVITY REPORT

27 February 2023

| Lic. No.: 11687 | Tundra Daly Sinclair Prov. HZNTL F5-36-8-29 (WPM) UWI: 107.05-36-008-29W1.00 Status: Licence Cancelled (CAN) Licence Cancelled: 27-Feb-2022 |
| --- | --- |
|  | Please note deleted UWI - Leg #1 |
| Lic. No.: 11693 | Tundra Virden Prov. HZNTL 3-11-9-25 (WPM) UWI: 102.03-11-009-25W1.00 Status: Licence Cancelled (CAN) Licence Cancelled: 27-Feb-2023 Please note deleted UWI - Leg #1 |
| Lic. No.: 11850 | Tundra Waskada Prov. HZNTL 2-18-2-25 (WPM) UWI: 102.02-18-002-25W1.00 Finished Drilling: 22-Feb-2023 Total Depth: 3207.00 m Production Casing: 139.70 mm @ 3207.00 m with 40.1 t Rig Released: 23-Feb-2023 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 23-Feb-2023 |
| Lic. No.: 11896 | Tundra Daly Sinclair HZNTL B9-7-10-28 (WPM) UWI: 104.09-07-010-28W1.00 - Leg #1 Status: Completing (COMP) Completing: 22-Feb-2023 |
| Lic. No.: 11899 | Corex Daly Sinclair HZNTL 15-19-10-27 (WPM) UWI: 100.15-19-010-27W1.00 Finished Drilling: 21-Feb-2023 Total Depth: 2237.00 m Production Casing: 139.70 mm @ 2237.00 m with 38.5 t Rig Released: 22-Feb-2023 |
|  | Status: Completing (COMP) Completing: 25-Feb-2023 |
| Lic. No.: 11900 | Corex Daly Sinclair HZNTL 16-19-10-27 (WPM) UWI: 102.16-19-010-27W1.00 Status: Completing (COMP) Completing: 24-Feb-2023 |
| Lic. No.: 11909 | Daly Unit No. 5 Prov. HZNTL A16-28-9-29 (WPM) UWI: 103.16-28-009-29W1.00 Status: Completing (COMP) Completing: 28-Feb-2023 |
| Lic. No.: 11910 | Tundra et al Daly Sinclair HZNTL B4-32-8-28 (WPM) UWI: 103.04-32-008-28W1.00 Status: Completing (COMP) Completing: 26-Feb-2023 |
| Lic. No.: 11914 | Tundra Daly Sinclair HZNTL A5-12-10-29 (WPM) UWI: 103.05-12-010-29W1.00 Status: Completing (COMP) Completing: 24-Feb-2023 |
| Lic. No.: 11915 | Tundra Daly Sinclair HZNTL 5-2-10-29 (WPM) UWI: 102.05-02-010-29W1.00 Spud Date: 24-Feb-2023 K.B. Elevation: 526.01 m Surface Casing: 244.50 mm @ 135.00 m with 6.5 t Status: Drilling Ahead (DR) Drilling Ahead: 27-Feb-2023 |
| Lic. No.: 11916 | Tundra Daly Sinclair HZNTL A5-2-10-29 (WPM) UWI: 103.05-02-010-29W1.00 Finished Drilling: 23-Feb-2023 Total Depth: 2142.00 m Production Casing: 139.70 mm @ 2142.00 m with 45.4 t Rig Released: 24-Feb-2023 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 24-Feb-2023 |
| Lic. No.: 11917 | Tundra Daly Sinclair HZNTL B4-21-9-29 (WPM) UWI: 104.04-21-009-29W1.00 Spud Date: 21-Feb-2023 K.B. Elevation: 539.30 m Surface Casing: 244.50 mm @ 136.00 m with 7.0 t Finished Drilling: 24-Feb-2023 Total Depth: 2206.00 m Production Casing: 139.70 mm @ 2206.00 m with 32.0 t Rig Released: 25-Feb-2023 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 25-Feb-2023 |
| Lic. No.: 11918 | Melita Pierson HZNTL A2-30-2-28 (WPM) UWI: 103.02-30-002-28W1.00 - Leg #1 UWI: 102.13-19-002-28W1.02 - Leg #2 UWI: 103.02-30-002-28W1.00 - Leg #1 Spud Date: 23-Feb-2023 K.B. Elevation: 465.30 m Surface Casing: 244.50 mm @ 149.00 m with 16.0 t Intermediate Casing: 177.80 mm @ 1123.00 m with 19.0 t Status: Drilling Ahead (DR) Drilling Ahead: 27-Feb-2023 |
| Lic. No.: 11920 | Tundra Regent Prov. HZNTL 3-11-4-22 (WPM) UWI: 100.03-11-004-22W1.00 Spud Date: 24-Feb-2023 K.B. Elevation: 502.58 m Surface Casing: 244.50 mm @ 145.00 m with 7.0 t Intermediate Casing: 177.80 mm @ 868.00 m with 16.0 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 27-Feb-2023 |
| Lic. No.: 11922 | Tundra Daly Sinclair HZNTL A16-22-9-29 (WPM) UWI: 103.16-22-009-29W1.00 Spud Date: 25-Feb-2023 K.B. Elevation: 537.60 m Surface Casing: 244.50 mm @ 135.00 m with 7.0 t Status: Drilling Ahead (DR) Drilling Ahead: 27-Feb-2023 |
| Lic. No.: 11926 | Tundra Waskada Prov. HZNTL A1-34-2-24 (WPM) UWI: 102.01-34-002-24W1.00 Licence Issued: 27-Feb-2023 Licensee: Tundra Oil and Gas Mineral Rights: Tundra Oil and Gas Contractor: Ensign Drilling Inc. - Rig# 10 Surface Location: 11D-35-2-24 Co-ords: 588.88 m S of N of Sec 35 666.01 m E of W of Sec 35 Grd Elev: 493.29 m Proj. TD: 2029.90 m (Mississippian) Area: Other Areas Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 11927 | Tundra et al Daly Sinclair Prov. HZNTL 16-5-10-29 (WPM) UWI: 100.16-05-010-29W1.00 Licence Issued: 27-Feb-2023 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 16D-6-10-29 Co-ords: 50.00 m S of N of Sec 6 177.06 m W of E of Sec 6 Grd Elev: 539.10 m Proj. TD: 2423.21 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |