| Economic Development, Investment, Trade & Natural Resources |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Petroleum Branch |  |  |  |  |  |  |  |  |
| 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 |  |  |  |  |  |  |  |  |
| T 204-945-6577 F 204-945-0586 |  |  |  |  |  |  |  |  |
| www.manitoba.ca |  |  |  |  |  |  |  |  |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| (January 1/23 -- Jul 31/23) |  |  |  |  |  |  |  |  |
| DRILLING ACTIVITY |  |  | To Jul 31/23 |  |  | To Aug 1/22 |  | 2022 Total |
| Drilling Licences Issued |  |  | 110 |  |  | 114 |  | 222 |
| Licences Cancelled |  |  | 5 |  |  | 3 |  | 11 |
| Vertical Wells Drilled |  |  | 0 |  |  | 2 |  | 7 |
| Horizontal Wells Drilled |  |  | 81 |  |  | 104 |  | 220 |
| Stratigraphic Test Holes Drilled |  |  | 1 |  |  | 0 |  | 1 |
| Wells Drilled - Total |  |  | 82 |  |  | 106 |  | 228 |
| No. of Metres Drilled |  | 184188 |  |  | 238944 |  |  | 503107 |
| Wells Re-entered |  |  | 0 |  |  | 3 |  | 7 |
| Wells Being Drilled |  |  | 2 |  |  | 3 |  | 0 |
| No. of Active Rigs |  |  | 3 |  |  | 5 |  | 0 |
| Wells Licenced but Not Spudded |  |  | 72 |  |  | 67 |  | 46 |
| Wells Completed as Potential Oil Wells |  |  | 80 |  |  | 102 |  | 221 |
| Wells Abandoned Dry |  |  | 1 |  |  | 0 |  | 1 |
| Wells Drilled but Not Completed |  |  | 1 |  |  | 2 |  | 0 |
| Other Completions |  |  | 0 |  |  | 2 |  | 3 |
| New Wells on Production |  |  | 0 |  |  | 0 |  | 0 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |  |
| Geophysical Programs Licenced |  |  | 0 |  |  | 0 |  | 0 |
| Licences Cancelled |  |  | 0 |  |  | 0 |  | 0 |
| Kilometers Licenced |  |  | 0 |  |  | 0.00 |  | 0 |
| Kilometers Run |  |  | 0 |  |  | 0 |  | 0 |
|  |  |  | 2023 |  |  | 2022 |  |  |
| OIL PRICES (Average) |  | $/M3 ¦ $/BBL |  |  | $/M3 ¦ $/BBL |  |  |  |
| Month of May |  | 558.07 | 88.68 |  | 850.72 | 135.18 |  |  |
| Month of June |  | 560.62 | 89.08 |  | 924.29 | 146.87 |  |  |
| OIL PRODUCTION (M3) |  |  | 2023 |  |  | 2022 |  |  |
| Month of June |  | 183,913.1 |  |  | 173,896.5 |  |  |  |

WEEKLY WELL ACTIVITY REPORT

31 July 2023

| Lic. No.: 11908 |  | Ewart Unit No. 18 HZNTL 5-1-9-29 (WPM) UWI: 102.05-01-009-29W1.00 Status: Completing (COMP) Completing: 30-Jul-2023 |  |  |
| --- | --- | --- | --- | --- |
| Lic. No.: 11939 |  | Tundra Daly Sinclair Prov. SWD DIR A13-10-9-28 (WPM) UWI: 104.13-10-009-28W1.00 Surface Casing: 244.50 mm @ 327.00 m with 14.5 t Status: Coring (COR) Coring: 31-Jul-2023 |  |  |
| Lic. No.: 11946 |  | Waskada Unit No. 22 HZNTL C5-15-1-25 (WPM) UWI: 105.05-15-001-25W1.00 |  |  |
|  |  | Status: Completing (COMP) Completing: 27-Jul-2023 |  |  |
| Lic. No.: 11952 |  | Waskada Unit No. 20 HZNTL B13-21-1-25 (WPM) UWI: 105.13-21-001-25W1.00 - Leg #1 UWI: 105.13-21-001-25W1.02 - Leg #2 Added UWI: 105.13-21-001-25W1.00 - Leg #1 Status: Junked and Abandoned (ABD J) Abandoned Junked: 25-Jul-2023 UWI: 105.13-21-001-25W1.02 - Leg #2 Kick-Off Date: 26-Jul-2023 Kick-Off Point: 1502.7 |  |  |
|  |  | Finished Drilling: 28-Jul-2023 Total Depth: 3214.00 m |  |  |
|  |  | Status: Running Casing (CSG) Running Casing: 31-Jul-2023 |  |  |
| Lic. No.: 11959 |  | Tundra Daly Sinclair Prov. HZNTL A1-10-10-29 (WPM) UWI: 102.01-10-010-29W1.00 Status: Completing (COMP) Completing: 29-Jul-2023 |  |  |
| Lic. No.: 11972 |  | Corex Daly Sinclair HZNTL C1-7-10-27 (WPM) UWI: 104.01-07-010-27W1.00 Finished Drilling: 23-Jul-2023 Total Depth: 2366.00 m Production Casing: 139.70 mm @ 2366.00 m with 47.0 t Rig Released: 28-Jul-2023 |  |  |
|  |  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 28-Jul-2023 |  |  |
| Lic. No.: 11973 |  | Corex Daly Sinclair HZNTL D1-7-10-27 (WPM) UWI: 105.01-07-010-27W1.00 Spud Date: 28-Jul-2023 K.B. Elevation: 493.86 m Surface Casing: 244.50 mm @ 146.00 m with 11.0 t Status: Drilling Ahead (DR) Drilling Ahead: 31-Jul-2023 |  |  |
| Lic. No.: 12002 |  | Tundra Pierson HZNTL A9-31-2-28 (WPM) UWI: 103.09-31-002-28W1.00 Licence Issued: 25-Jul-2023 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign Drilling Inc. - Rig# 10 Surface Location: 14A-36-2-29 Co-ords: 445.00 m S of N of Sec 36 744.11 m E of W of Sec 36 Grd Elev: 464.40 m Proj. TD: 3252.42 m (Triassic) Field: Pierson Classification: Non Confidential Development Status: Location(LOC) |  |  |
|  |  | 102.09-31-002-28W1.00 - Leg #1 Please note revised UWI |  |  |
| Lic. No.: 12003 |  | Daly Unit No. 4 HZNTL B2-35-9-28 (WPM) UWI: 104.02-35-009-28W1.00 Licence Issued: 26-Jul-2023 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 1D-35-9-28 Co-ords: 239.38 m N of S of Sec 34 201.02 m W of E of Sec 34 Grd Elev: 498.16 m Proj. TD: 1938.42 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |  |  |
| Lic. No.: 12004 |  | Daly Unit No. 4 HZNTL C2-35-9-28 (WPM) UWI: 105.02-35-009-28W1.00 Licence Issued: 26-Jul-2023 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 1D-34-9-28 Co-ords: 219.41 m N of S of Sec 34 201.00 m W of E of Sec 34 Grd Elev: 498.11 m Proj. TD: 1915.15 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |  |  |
| Lic. No.: 12005 |  | North Virden Scallion Unit No. 1 HZNTL A11-11-11-26 (WPM) UWI: 103.11-11-011-26W1.00 UWI: 103.11-11-011-26W1.02 Licence Issued: 28-Jul-2023 Licensee: Corex Resources Ltd Mineral Rights: Corex Resources Ltd Contractor: Ensign Drilling Inc. - Rig# 14 Surface Location: 6B-14-11-26 Co-ords: 588.00 m N of S of Sec 14 514.00 m E of W of Sec 14 Grd Elev: 453.12 m Proj. TD: 1864.78 m (Mississippian) Field: Virden Classification: Non Confidential Development Status: Location(LOC) |  |  |
| Lic. No.: 12006 |  | North Virden Scallion Unit No. 1 HZNTL A4-33-11-26 (WPM) UWI: 103.04-33-011-26W1.00 Licence Issued: 28-Jul-2023 Licensee: Corex Resources Ltd Mineral Rights: Corex Resources Ltd Contractor: Ensign Drilling Inc. - Rig# 14 Surface Location: 3B-28-11-26 Co-ords: 160.19 m N of S of Sec 28 504.97 m E of W of Sec 28 Grd Elev: 459.10 m Proj. TD: 2040.75 m (Mississippian) Field: Virden Classification: Non Confidential Development Status: Location(LOC) |  |  |
| Lic. No.: 12007 |  | Corex Daly Sinclair Prov. HZNTL 3-8-10-27 (WPM) UWI: 100.03-08-010-27W1.00 Licence Issued: 28-Jul-2023 Licensee: Corex Resources Ltd Mineral Rights: Corex Resources Ltd Contractor: Ensign Drilling Inc. - Rig# 14 Surface Location: 4B-17-10-27 Co-ords: 140.00 m N of S of Sec 17 150.00 m E of W of Sec 17 Grd Elev: 487.60 m Proj. TD: 2448.30 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |  |  |
| Lic. No.: 12008 |  | Corex Daly Sinclair Prov. HZNTL 4-8-10-27 (WPM) UWI: 100.04-08-010-27W1.00 Licence Issued: 28-Jul-2023 Licensee: Corex Resources Ltd Mineral Rights: Corex Resources Ltd Contractor: Ensign Drilling Inc. - Rig# 14 Surface Location: 4B-17-10-27 Co-ords: 140.00 m N of S of Sec 17 110.00 m E of W of Sec 17 Grd Elev: 488.27 m Proj. TD: 2284.03 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |  |  |