| Economic Development, Investment, Trade & Natural Resources |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Petroleum Branch |  |  |  |  |  |  |  |  |
| 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 |  |  |  |  |  |  |  |  |
| T 204-945-6577 F 204-945-0586 |  |  |  |  |  |  |  |  |
| www.manitoba.ca |  |  |  |  |  |  |  |  |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| (January 1/23 -- Aug 14/23 ) |  |  |  |  |  |  |  |  |
| DRILLING ACTIVITY |  |  | To Aug 14/23 |  |  | To Aug 15/22 |  | 2022 Total |
| Drilling Licences Issued |  |  | 116 |  |  | 131 |  | 222 |
| Licences Cancelled |  |  | 5 |  |  | 3 |  | 11 |
| Vertical Wells Drilled |  |  | 1 |  |  | 2 |  | 7 |
| Horizontal Wells Drilled |  |  | 87 |  |  | 117 |  | 220 |
| Stratigraphic Test Holes Drilled |  |  | 1 |  |  | 0 |  | 1 |
| Wells Drilled - Total |  |  | 89 |  |  | 119 |  | 228 |
| No. of Metres Drilled |  | 198668 |  |  | 269128 |  |  | 503107 |
| Wells Re-entered |  |  | 0 |  |  | 3 |  | 7 |
| Wells Being Drilled |  |  | 3 |  |  | 1 |  | 0 |
| No. of Active Rigs |  |  | 4 |  |  | 4 |  | 0 |
| Wells Licenced but Not Spudded |  |  | 70 |  |  | 73 |  | 46 |
| Wells Completed as Potential Oil Wells |  |  | 88 |  |  | 115 |  | 221 |
| Wells Abandoned Dry |  |  | 1 |  |  | 0 |  | 1 |
| Wells Drilled but Not Completed |  |  | 0 |  |  | 2 |  | 0 |
| Other Completions |  |  | 0 |  |  | 2 |  | 3 |
| New Wells on Production |  |  | 0 |  |  | 0 |  | 0 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |  |
| Geophysical Programs Licenced |  |  | 0 |  |  | 0 |  | 0 |
| Licences Cancelled |  |  | 0 |  |  | 0 |  | 0 |
| Kilometers Licenced |  |  | 0 |  |  | 0.00 |  | 0 |
| Kilometers Run |  |  | 0 |  |  | 0 |  | 0 |
|  |  |  | 2023 |  |  | 2022 |  |  |
| OIL PRICES (Average) |  | $/M3 ¦ $/BBL |  |  | $/M3 ¦ $/BBL |  |  |  |
| Month of June |  | 560.62 | 89.08 |  | 924.29 | 146.87 |  |  |
| Month of July |  | 661.17 | 105.06 |  | 776.40 | 123.37 |  |  |
| OIL PRODUCTION (M3) |  |  | 2023 |  |  | 2022 |  |  |
| Month of July |  | 188,171.3 |  |  | 177,441.8 |  |  |  |

WEEKLY WELL ACTIVITY REPORT

14 August 2023

| Lic. No.: 11713 | Shiffoil Pierson HZNTL 10-17-1-27 (WPM) UWI: 100.10-17-001-27W1.00 Please note revision to Report dated 16-May-2022 Drilling Contractor: Betts Drilling Ltd. – Rig #03 Spud Date: 12-Aug-2023 K.B. Elevation: 457.50 m Surface Casing: 244.50 mm @ 151.00 m with 12.8 t |
| --- | --- |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 14-Aug-2023 |
| Lic. No.: 11939 | Tundra Daly Sinclair Prov. SWD DIR A13-10-9-28 (WPM) UWI: 104.13-10-009-28W1.00 Rig Released: 08-Aug-2023 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 08-Aug-2023 |
| Lic. No.: 11942 | Tundra Daly Sinclair HZNTL F16-19-8-28 (WPM) UWI: 107.16-19-008-28W1.00 Spud Date: 14-Aug-2023 K.B. Elevation: 516.14 m |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 14-Aug-2023 |
| Lic. No.: 11945 | Tundra Whitewater HZNTL 15-16-3-21 (WPM) UWI: 100.15-16-003-21W1.00 - Leg#1 UWI: 103.14-16-003-21W1.02 - Leg#2 UWI: 100.15-16-003-21W1.00 - Leg#1 Spud Date: 08-Aug-2023 K.B. Elevation: 504.36 m Surface Casing: 244.50 mm @ 140.00 m with 7.0 t Intermediate Casing: 177.80 mm @ 886.00 m with 15.0 t Finished Drilling: 12-Aug-2023 Total Depth: 1754.00 m UWI: 103.14-16-003-21W1.02 - Leg#2 Kick-Off Date: 12-Aug-2023 Kick-Off Point: 908.00 m Finished Drilling: 14-Aug-2023 Total Depth: 1781.00 m UWI: 100.15-16-003-21W1.00 - Leg#1 UWI: 103.14-16-003-21W1.02 - Leg#2 Rig Released: 14-Aug-2023 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 14-Aug-2023 |
| Lic. No.: 11952 | Waskada Unit No. 20 HZNTL B13-21-1-25 (WPM) UWI: 105.13-21-001-25W1.00 Status: Completing (COMP) Completing: 14-Aug-2023 |
| Lic. No.: 11969 | Corex Lenore 13-1-11-25 (WPM) UWI: 100.13-01-011-25W1.00 Status: Moving In Rotary Tools (MIRT) Moving In Rotary Tools: 14-Aug-2023 |
| Lic. No.: 11970 | Corex Daly Sinclair HZNTL B2-7-10-27 (WPM) UWI: 103.02-07-010-27W1.00 Status: Completing (COMP) Completing: 10-Aug-2023 |
| Lic. No.: 11971 | Corex Daly Sinclair HZNTL B1-7-10-27 (WPM) UWI: 103.01-07-010-27W1.00 Status: Completing (COMP) Completing: 11-Aug-2023 |
| Lic. No.: 11972 | Corex Daly Sinclair HZNTL C1-7-10-27 (WPM) UWI: 104.01-07-010-27W1.00 Status: Completing (COMP) Completing: 13-Aug-2023 |
| Lic. No.: 11975 | Daly Unit No. 1 HZNTL B2-9-10-28 (WPM) UWI: 104.02-09-010-28W1.00 Status: Completing (COMP) Completing: 08-Aug-2023 |
| Lic. No.: 11976 | Corex Daly Sinclair Prov. HZNTL C3-10-10-28 (WPM) UWI: 105.03-10-010-28W1.00 Status: Completing (COMP) Completing: 09-Aug-2023 |
| Lic. No.: 11982 | Corex Daly Sinclair HZNTL 9-17-10-27 (WPM) UWI: 102.09-17-010-27W1.00 Production Casing: 139.70 mm @ 2328.00 m with 44.5 t Rig Released: 08-Aug-2023 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 08-Aug-2023 |
| Lic. No.: 11983 | Corex Daly Sinclair HZNTL 16-17-10-27 (WPM) UWI: 100.16-17-010-27W1.00 Spud Date: 09-Aug-2023 K.B. Elevation: 493.30 m Surface Casing: 244.50 mm @ 142.00 m with 12.0 t Finished Drilling: 13-Aug-2023 Total Depth: 2438.00 m Production Casing: 139.70 mm @ 2438.00 m with 48.0 t Rig Released: 14-Aug-2023 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 14-Aug-2023 |
| Lic. No.: 11984 | Whitewater Unit No. 2 HZNTL 2-2-3-21 (WPM) UWI: 102.02-02-003-21W1.00 - Leg #1 UWI: 102.02-02-003-21W1.02 - Leg #2 UWI: 102.02-02-003-21W1.02 - Leg #2 Finished Drilling: 08-Aug-2023 Total Depth: 1976.00 m UWI: 102.02-02-003-21W1.00 - Leg #1 UWI: 102.02-02-003-21W1.02 - Leg #2 Rig Released: 09-Aug-2023 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 09-Aug-2023 |
| Lic. No.: 12003 | Daly Unit No. 4 HZNTL B2-35-9-28 (WPM) UWI: 104.02-35-009-28W1.00 Spud Date: 12-Aug-2023 K.B. Elevation: 502.36 m Surface Casing: 244.50 mm @ 144.00 m with 7.0 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 14-Aug-2023 |
| Lic. No.: 12004 | Daly Unit No. 4 HZNTL C2-35-9-28 (WPM) UWI: 105.02-35-009-28W1.00 Spud Date: 08-Aug-2023 K.B. Elevation: 502.31 m Surface Casing: 244.50 mm @ 135.00 m with 7.5 t Finished Drilling: 11-Aug-2023 Total Depth: 1914.00 m Production Casing: 139.70 mm @ 1914.00 m with 30.6 t Rig Released: 12-Aug-2023 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 12-Aug-2023 |
| Lic. No.: 12009 | North Pierson Unit No. 3 Prov. HZNTL 7-18-3-29 (WPM) UWI: 100.07-18-003-29W1.00 Licence Issued: 09-Aug-2023 Licensee: Burgess Creek Exploration Inc. Mineral Rights: Burgess Creek Exploration Inc. Contractor: Betts Drilling Ltd. - Rig# 4 Surface Location: 14D-6-3-29 Co-ords: 100.00 m S of N of Sec 6 690.00 m E of W of Sec 6 Grd Elev: 485.66 m Proj. TD: 3469.65 m (Mississippian) Field: Pierson Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12010 | Corex Daly Sinclair Prov. HZNTL 3-6-10-27 (WPM) UWI: 100.03-06-010-27W1.00 Licence Issued: 11-Aug-2023 Licensee: Corex Resources Ltd Mineral Rights: Corex Resources Ltd Contractor: Ensign Drilling Inc. - Rig# 14 Surface Location: 11B-6-10-27 Co-ords: 660.00 m S of N of Sec 6 410.00 m E of W of Sec 6 Grd Elev: 493.47 m Proj. TD: 1481.09 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12011 | Corex Daly Sinclair Prov. HZNTL 4-6-10-27 (WPM) UWI: 102.04-06-010-27W1.00 Licence Issued: 11-Aug-2023 Licensee: Corex Resources Ltd Mineral Rights: Corex Resources Ltd Contractor: Ensign Drilling Inc. - Rig# 14 Surface Location: 12A-6-10-27 Co-ords: 660.00 m S of N of Sec 6 370.00 m E of W of Sec 6 Grd Elev: 491.71 m Proj. TD: 1601.30 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12012 | Tundra Daly Sinclair HZNTL 4-35-9-29 (WPM) UWI: 100.04-35-009-29W1.00 Licence Issued: 11-Aug-2023 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 5B-36-9-29 Co-ords: 458.41 m N of S of Sec 36 60.00 m E of W of Sec 36 Grd Elev: 522.81 m Proj. TD: 2334.90 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12013 | Tundra Daly Sinclair HZNTL A4-14-10-29 (WPM) UWI: 102.04-14-010-29W1.00 Licence Issued: 13-Aug-2023 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 1D-14-10-29 Co-ords: 240.64 m N of S of Sec 14 65.00 m W of E of Sec 14 Grd Elev: 524.15 m Proj. TD: 2161.93 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12014 | Tundra Daly Sinclair HZNTL 5-14-10-29 (WPM) UWI: 100.05-14-010-29W1.00 Licence Issued: 13-Aug-2023 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 8A-14-10-29 Co-ords: 494.48 m N of S of Sec 14 60.00 m W of E of Sec 14 Grd Elev: 522.82 m Proj. TD: 2153.57 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |