| Economic Development, Investment, Trade & Natural Resources |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Petroleum Branch |  |  |  |  |  |  |  |  |
| 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 |  |  |  |  |  |  |  |  |
| T 204-945-6577 F 204-945-0586 |  |  |  |  |  |  |  |  |
| www.manitoba.ca |  |  |  |  |  |  |  |  |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| (January 1/23 -- Aug 21/23 ) |  |  |  |  |  |  |  |  |
| DRILLING ACTIVITY |  |  | To Aug 21/23 |  |  | To Aug 22/22 |  | 2022 Total |
| Drilling Licences Issued |  |  | 117 |  |  | 138 |  | 222 |
| Licences Cancelled |  |  | 5 |  |  | 3 |  | 11 |
| Vertical Wells Drilled |  |  | 2 |  |  | 2 |  | 7 |
| Horizontal Wells Drilled |  |  | 90 |  |  | 121 |  | 220 |
| Stratigraphic Test Holes Drilled |  |  | 1 |  |  | 0 |  | 1 |
| Wells Drilled - Total |  |  | 93 |  |  | 123 |  | 228 |
| No. of Metres Drilled |  | 205349 |  |  | 278596 |  |  | 503107 |
| Wells Re-entered |  |  | 0 |  |  | 3 |  | 7 |
| Wells Being Drilled |  |  | 3 |  |  | 4 |  | 0 |
| No. of Active Rigs |  |  | 3 |  |  | 4 |  | 0 |
| Wells Licenced but Not Spudded |  |  | 67 |  |  | 73 |  | 46 |
| Wells Completed as Potential Oil Wells |  |  | 91 |  |  | 121 |  | 221 |
| Wells Abandoned Dry |  |  | 2 |  |  | 0 |  | 1 |
| Wells Drilled but Not Completed |  |  | 0 |  |  | 0 |  | 0 |
| Other Completions |  |  | 0 |  |  | 2 |  | 3 |
| New Wells on Production |  |  | 0 |  |  | 0 |  | 0 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |  |
| Geophysical Programs Licenced |  |  | 0 |  |  | 0 |  | 0 |
| Licences Cancelled |  |  | 0 |  |  | 0 |  | 0 |
| Kilometers Licenced |  |  | 0 |  |  | 0.00 |  | 0 |
| Kilometers Run |  |  | 0 |  |  | 0 |  | 0 |
|  |  |  | 2023 |  |  | 2022 |  |  |
| OIL PRICES (Average) |  | $/M3 ¦ $/BBL |  |  | $/M3 ¦ $/BBL |  |  |  |
| Month of June |  | 560.62 | 89.08 |  | 924.29 | 146.87 |  |  |
| Month of July |  | 661.17 | 105.06 |  | 776.40 | 123.37 |  |  |
| OIL PRODUCTION (M3) |  |  | 2023 |  |  | 2022 |  |  |
| Month of July |  | 188,171.3 |  |  | 177,441.8 |  |  |  |

WEEKLY WELL ACTIVITY REPORT

21 August 2023

| Lic. No.: 11713 | Shiffoil Pierson HZNTL 10-17-1-27 (WPM) UWI: 100.10-17-001-27W1.00 Finished Drilling: 16-Aug-2023 Total Depth: 1746.00 m Production Casing: 139.70 mm @ 1736.50 m with 40.0 t Rig Released: 17-Aug-2023 |
| --- | --- |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 17-Aug-2023 |
| Lic. No.: 11939 | Tundra Daly Sinclair Prov. SWD DIR A13-10-9-28 (WPM) UWI: 104.13-10-009-28W1.00 Status: Completing (COMP) Completing: 18-Aug-2023 |
| Lic. No.: 11941 | Tundra Daly Sinclair HZNTL C9-19-8-28 (WPM) UWI: 104.09-19-008-28W1.00 Spud Date: 19-Aug-2023 K.B. Elevation: 515.88 m Surface Casing: 244.50 mm @ 135.00 m with 7.0 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 21-Aug-2023 |
| Lic. No.: 11942 | Tundra Daly Sinclair HZNTL F16-19-8-28 (WPM) UWI: 107.16-19-008-28W1.00 Surface Casing: 244.50 mm @ 135.00 m with 6.5 t Finished Drilling: 18-Aug-2023 Total Depth: 2334.00 m Production Casing: 139.70 mm @ 2334.00 m with 30.0 t Rig Released: 19-Aug-2023 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 19-Aug-2023 |
| Lic. No.: 11943 | Tundra Daly Sinclair HZNTL A5-18-8-28 (WPM) 102.05-18-008-28W1.00 102.05-18-008-28W1.00 - Leg #1 Please note revised UWI |
| Lic. No.: 11945 | Tundra Whitewater HZNTL 15-16-3-21 (WPM) UWI: 100.15-16-003-21W1.00 UWI: 103.14-16-003-21W1.02 Status: Completing (COMP) Completing: 22-Aug-2023 |
| Lic. No.: 11947 | Corex Virden HZNTL HZNTL 13-9-12-26 (WPM) UWI: 100.13-09-012-26W1.00 Spud Date: 18-Aug-2023 K.B. Elevation: 466.78 m Surface Casing: 244.50 mm @ 144.00 m with 11.0 t Intermediate Casing: 177.80 mm @ 756.00 m with 12.5 t |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 21-Aug-2023 |
| Lic. No.: 11969 | Corex Lenore 13-1-11-25 (WPM) UWI: 100.13-01-011-25W1.00 Spud Date: 15-Aug-2023 K.B. Elevation: 453.44 m Surface Casing: 244.50 mm @ 104.00 m with 8.5 t Finished Drilling: 17-Aug-2023 Total Depth: 678.00 m Rig Released: 18-Aug-2023 |
|  | Status: Abandoned Dry (ABD D) Abandoned Dry: 18-Aug-2023 |
| Lic. No.: 11973 | Corex Daly Sinclair HZNTL D1-7-10-27 (WPM) UWI: 105.01-07-010-27W1.00 Status: Completing (COMP) Completing: 15-Aug-2023 |
| Lic. No.: 11978 | Tundra Daly Sinclair SWD DIR 8-28-9-29 (WPM) UWI: 102.08-28-009-29W1.00 Spud Date: 16-Aug-2023 K.B. Elevation: 537.44 m Surface Casing: 244.50 mm @ 210.00 m with 11.0 t Intermediate Casing: 177.80 mm @ 1473.00 m with 22.2 t |
|  | Status: Cementing Casing (CMT CSG) Cementing Casing: 21-Aug-2023 |
| Lic. No.: 11982 | Corex Daly Sinclair HZNTL 9-17-10-27 (WPM) UWI: 102.09-17-010-27W1.00 Status: Completing (COMP) Completing: 15-Aug-2023 |
| Lic. No.: 11983 | Corex Daly Sinclair HZNTL 16-17-10-27 (WPM) UWI: 100.16-17-010-27W1.00 Status: Completing (COMP) Completing: 18-Aug-2023 |
| Lic. No.: 11984 | Whitewater Unit No. 2 HZNTL 2-2-3-21 (WPM) UWI: 102.02-02-003-21W1.00 - Leg #1 UWI: 102.02-02-003-21W1.02 - Leg #2 Status: Completing (COMP) Completing: 15-Aug-2023 |
| Lic. No.: 12003 | Daly Unit No. 4 HZNTL B2-35-9-28 (WPM) UWI: 104.02-35-009-28W1.00 Finished Drilling: 15-Aug-2023 Total Depth: 1923.00 m Production Casing: 139.70 mm @ 1923.00 m with 30.1 t Rig Released: 15-Aug-2023 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 15-Aug-2023 |
| Lic. No.: 12015 | East Manson Unit No. 3 Prov. HZNTL 12-20-13-28 (WPM) UWI: 102.12-20-013-28W1.00 Licence Issued: 21-Aug-2023 Licensee: Tundra Oil and Gas Mineral Rights: Tundra Oil and Gas Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 13B-21-13-28 Co-ords: 239.90 m S of N of Sec 21 100.00 m E of W of Sec 21 Grd Elev: 488.42 m Proj. TD: 2312.52 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |