| Economic Development, Investment, Trade & Natural Resources |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Petroleum Branch |  |  |  |  |  |  |  |  |
| 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 |  |  |  |  |  |  |  |  |
| T 204-945-6577 F 204-945-0586 |  |  |  |  |  |  |  |  |
| www.manitoba.ca |  |  |  |  |  |  |  |  |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| (January 1/24 -- March 04/24 ) |  |  |  |  |  |  |  |  |
| DRILLING ACTIVITY |  |  | To Mar 04/24 |  |  | To Mar 06/23 |  | 2023 Total |
| Drilling Licences Issued |  |  | 26 |  |  | 45 |  | 187 |
| Licences Cancelled |  |  | 4 |  |  | 2 |  | 8 |
| Vertical Wells Drilled |  |  | 0 |  |  | 0 |  | 3 |
| Horizontal Wells Drilled |  |  | 38 |  |  | 48 |  | 159 |
| Stratigraphic Test Holes Drilled |  |  | 0 |  |  | 1 |  | 1 |
| Wells Drilled - Total |  |  | 38 |  |  | 49 |  | 162 |
| No. of Metres Drilled |  | 90916 |  |  | 108718 |  |  | 366427 |
| Wells Re-entered |  |  | 1 |  |  | 0 |  | 1 |
| Wells Being Drilled |  |  | 1 |  |  | 2 |  | 0 |
| No. of Active Rigs |  |  | 1 |  |  | 4 |  | 0 |
| Wells Licenced but Not Spudded |  |  | 48 |  |  | -16 |  | 17 |
| Wells Completed as Potential Oil Wells |  |  | 37 |  |  | 45 |  | 162 |
| Wells Abandoned Dry |  |  | 1 |  |  | 1 |  | 3 |
| Wells Drilled but Not Completed |  |  | 1 |  |  | 2 |  | 0 |
| Other Completions |  |  | 0 |  |  | 1 |  | 1 |
| New Wells on Production |  |  | 17 |  |  | 0 |  | 0 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |  |
| Geophysical Programs Licenced |  |  | 0 |  |  | 1 |  | 1 |
| Licences Cancelled |  |  | 0 |  |  | 0 |  | 0 |
| Kilometers Licenced |  |  | 0.00 |  |  | 169.74 |  | 169.74 |
| Kilometers Run |  |  | 0.00 |  |  | 0.00 |  | 0.00 |
|  |  |  | 2024 |  |  | 2023 |  |  |
| OIL PRICES (Average) |  | $/M3 ¦ $/BBL |  |  | $/M3 ¦ $/BBL |  |  |  |
| Month of January |  | 535.7 | 85.11 |  | 615.5 | 97.81 |  |  |
| Month of February |  | 564.1 | 89.64 |  | 590.4 | 93.81 |  |  |
| OIL PRODUCTION (M3) |  |  | 2024 |  |  | 2023 |  |  |
| Month of February |  | 175,996.6 |  |  | 180,652.7 |  |  |  |

WEEKLY WELL ACTIVITY REPORT

04 March 2024

| Lic. No.: 11935 | Waskada Unit No. 16 HZNTL B10-5-2-25 (WPM) UWI: 104.10-05-002-25W1.00 Status: Completing (COMP) Completing: 16-Feb-2024 |
| --- | --- |
| Lic. No.: 11949 | Virden Roselea Unit No. 3 HZNTL 4-11-10-26 (WPM) UWI: 102.04-11-010-26W1.02 - Leg #1 UWI: 102.04-11-010-26W1.02 - Leg #2 |
|  | Status: Completing (COMP) Completing: 23-Feb-2024 |
| Lic. No.: 11951 | Virden Roselea Unit No. 1 HZNTL 15-26-10-26 (WPM) UWI: 102.15-26-010-26W1.02 - Leg #1 UWI: 102.15-26-010-26W1.02 - Leg #2 UWI: 102.15-26-010-26W1.02 - Leg #1 Surface Casing: 244.50 mm @ 142.00 m with 11.0 t Intermediate Casing: 177.80 mm @ 740.00 m with 17.0 t Finished Drilling: 24-Feb-2024 Total Depth: 1497.00 m UWI: 102.15-26-010-26W1.02 - Leg #2 |
|  | Kick-Off Date: 25-Feb-2024 Kick-Off Point: 764 |
|  | Finished Drilling: 26-Feb-2024 Total Depth: 1471.00 m UWI: 102.15-26-010-26W1.02 - Leg #1 UWI: 102.15-26-010-26W1.02 - Leg #2 |
|  | Rig Released: 26-Feb-2024 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 26-Feb-2024 |
| Lic. No.: 11986 | Tundra Daly Sinclair HZNTL B13-23-9-29 (WPM) UWI: 104.13-23-009-29W1.00 Status: Completing (COMP) Completing: 21-Feb-2024 |
| Lic. No.: 12037 | North Virden Scallion Unit No. 1 HZNTL B5-33-11-26 (WPM) UWI: 104.05-33-011-26W1.00 Spud Date: 27-Feb-2024 K.B. Elevation: 465.48 m Surface Casing: 244.50 mm @ 151.00 m with 15.0 t Intermediate Casing: 177.80 mm @ 793.00 m with 18.0 t |
|  | Finished Drilling: 03-Mar-2024 Total Depth: 1713.00 m Rig Released: 03-Mar-2024 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 03-Mar-2024 |
| Lic. No.: 12040 | Tundra Waskada HZNTL 11-1-2-25 (WPM) UWI: 100.11-01-002-25W1.00 Status: Completing (COMP) Completing: 20-Feb-2024 |
| Lic. No.: 12052 | Corex Daly Sinclair Prov. HZNTL A3-6-10-27 (WPM) UWI: 102.03-06-010-27W1.00 Status: Completing (COMP) Completing: 18-Feb-2024 |
| Lic. No.: 12053 | Corex Daly Sinclair Prov. HZNTL A4-6-10-27 (WPM) UWI: 103.04-06-010-27W1.00 Status: Completing (COMP) Completing: 17-Feb-2024 |
| Lic. No.: 12055 | Tundra Daly Sinclair HZNTL A1-22-10-28 (WPM) UWI: 102.01-22-010-28W1.00 Spud Date: 22-Feb-2024 K.B. Elevation: 513.82 Surface Casing: 244.50 mm @ 141.0 m with 6.5 t Finished Drilling: 26-Feb-2024 Production Casing: 139.70 mm @ 2279.00 m with 35.0 t Total Depth: 2279.00 m Rig Released: 26-Feb-2024 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 26-Feb-2024 |
| Lic. No.: 12059 | Tundra Daly Sinclair HZNTL B4-14-10-29 (WPM) UWI: 103.04-14-010-29W1.00 Spud Date: 25-Feb-2024 K.B. Elevation: 528.28 m Surface Casing: 244.50 mm @ 135.00 m with 8.0 t Production Casing: Run Finished Drilling: 28-Feb-2024 Total Depth: 2175.00 m Rig Released: 29-Feb-2024 Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 29-Feb-2024 |
| Lic. No.: 12060 | Tundra Daly Sinclair HZNTL A5-14-10-29 (WPM) UWI: 102.05-14-010-29W1.00 Status: Completing (COMP) Completing: 27-Feb-2024 |
| Lic. No.: 12069 | Tundra Daly Sinclair A4-20-10-28 (WPM) UWI: 104.04-20-010-28W1.00 |
|  | 104.04-20-010-28W1.00 - Leg #1 Please note revised UWI |
|  | Spud Date: 27-Feb-2024 K.B. Elevation: 520.12 m Surface Casing: 244.50 mm @ 137.00 m with 6.5 t Finished Drilling: 27-Feb-2024 Total Depth: 137.00 m Rig Released: 29-Feb-2024 Status: Abandoned Junked (ABD J) |
| Lic. No.: 12072 | Tundra Daly Sinclair HZNTL B1-27-9-29 (WPM) UWI: 104.01-27-009-29W1.00 Spud Date: 24-Feb-2024 K.B. Elevation: 535.57 m Surface Casing: 244.50 mm @ 142.00 m with 7.0 t Finished Drilling: 27-Feb-2024 Total Depth: 2337.00 m Production Casing: 139.70 mm @ 2337.00 m with 37.0 t Rig Released: 28-Feb-2024 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 28-Feb-2024 |
| Lic. No.: 12075 | Tundra Daly Sinclair HZNTL B9-1-10-29 (WPM) UWI: 103.09-01-010-29W1.00 Spud Date: 29-Feb-2024 K.B. Elevation: 525.49 m Surface Casing: 244.50 mm @ 136.00 m with 8.0 t Finished Drilling: 03-Mar-2024 Total Depth: 2322.00 m Status: Running Casing (CSG) Running Casing: 04-Mar-2024 |
| Lic. No.: 12076 | Tundra Daly Sinclair HZNTL D12-18-8-28 (WPM) UWI: 105.12-18-008-28W1.00 Status: Completing (COMP) Completing: 17-Feb-2024 |
| Lic. No.: 12078 | Virden Roselea Unit No. 3 HZNTL 5-10-10-26 (WPM) UWI: 102.05-10-010-26W1.00 Status: Completing (COMP) Completing: 29-Feb-2024 |
| Lic. No.: 12079 | Virden Roselea Unit No. 3 HZNTL A12-10-10-26 (WPM) UWI: 103.12-10-010-26W1.00 Status: Completing (COMP) Completing: 26-Feb-2024 |
| Lic. No.: 12080 | Ewart Unit No. 15 HZNTL C9-33-8-28 (WPM) UWI: 104.09-33-008-28W1.00 Status: Completing (COMP) Completing: 14-Feb-2024 |
| Lic. No.: 12082 | Tundra Waskada HZNTL 14-14-2-25 (WPM) UWI: 102.14-14-002-25W1.00 Status: Completing (COMP) Completing: 23-Feb-2024 |
| Lic. No.: 12084 | Tundra Daly Sinclair HZNTL A14-15-10-29 (WPM) UWI: 102.14-15-010-29W1.00 Status: Completing (COMP) Completing: 22-Feb-2024 |
| Lic. No.: 12086 | Shiffoil Pierson HZNTL A10-32-1-28 (WPM) UWI: 102.10-32-001-28W1.00 Spud Date: 27-Feb-2024 K.B. Elevation: 464.00 m Surface Casing: 244.50 mm @ 155.00 m with 15.5 t Intermediate Casing: 139.70 mm @ 1107.00 m with 48.5 t Finished Drilling: 02-Mar-2024 Total Depth: 1703.00 m Rig Released: 03-Mar-2024 |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 03-Mar-2024 |
| Lic. No.: 12087 | Corex Daly Sinclair HZNTL D1-8-9-28 (WPM) UWI: 105.01-08-009-28W1.00 Status: Completing (COMP) Completing: 16-Feb-2024 |
| Lic. No.: 12089 | Tundra et al Daly Sinclair Prov. HZNTL B9-20-8-28 (WPM) UWI: 104.09-20-008-28W1.00 Status: Completing (COMP) Completing: 19-Feb-2024 |
| Lic. No.: 12091 | Tundra Daly Sinclair HZNTL A1-28-10-28 (WPM) UWI: 102.01-28-010-28W1.00 Status: Completing (COMP) Completing: 26-Feb-2024 |
| Lic. No.: 12098 | Tundra Daly Sinclair HZNTL C4-19-10-28 (WPM) UWI: 104.04-19-010-28W1.00 - Leg #1 Spud Date: 29-Feb-2024 K.B. Elevation: 519.93 m Surface Casing: 244.50 mm @ 141.00 m with 6.5 t Finished Drilling: 02-Mar-2024 Total Depth: 2259.00 m Production Casing: 139.70 mm @ 2259.00 m with 36.0 t Status: Waiting On Orders (WOO) Waiting On Orders: 04-Mar-2024. |
| Lic. No.: 12100 | Corex Daly Sinclair Prov. SWD A2-11-10-28 (WPM) UWI: 102.02-11-010-28W1.00 Status: Moving In Rotary Tools (MIRT) Moving In Rotary Tools: 04-Mar-2024 |
| Lic. No.: 12102 | Tundra South Reston Prov. HZNTL B8-14-6-27 (WPM) UWI: 102.03-13-006-27W1.00 Spud Date: 29-Feb-2024 K.B. Elevation: 443.04 m |
|  | Status: Drilling Ahead (DR) Drilling Ahead: 04-Mar-2024 |
| Lic. No.: 12108 | Goodlands Unit No. 3 Prov. HZNTL B3-16-1-24 (WPM) UWI: 103.03-16-001-24W1.00 Licence Issued: 01-Mar-2024 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 10 Surface Location: 1D-18-1-24 Co-ords: 225.00 m N of S of Sec 18 60.00 m W of E of Sec 18 Grd Elev: 476.10 m Proj. TD: 3217.40 m (Triassic) Field: Waskada Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12109 | Tundra Daly Sinclair HZNTL D4-19-10-28 (WPM) UWI: 105.04-19-010-28W1.00 Licence Issued: 04-Mar-2024 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 4C-20-10-28 Co-ords: 283.94 m N of S of Sec 20 55.00 m E of W of Sec 20 Grd Elev: 516.15 m Proj. TD: 2234.66 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12110 | Daly Unit No. 4 HZNTL A1-2-10-28 (WPM) UWI: 103.01-02-010-28W1.00 Licence Issued: 04-Mar-2024 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign Drilling Inc. - Rig# 10 Surface Location: 1A-35-9-28 Co-ords: 51.61 m N of S of Sec 35 125.67 m W of E of Sec 35 Grd Elev: 492.69 m Proj. TD: 2370.97 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12111 | Tundra Daly Sinclair Prov. HZNTL A16-18-10-27 (WPM) UWI:102.16-18-010-27W1.00 Licence Issued: 04-Mar-2024 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign - Trinidad Drilling Inc. - Rig# 9 Surface Location: 16C-7-10-27 Co-ords: 60.00 m S of N of Sec 7 224.92 m W of E of Sec 7 Grd Elev: 489.68 m Proj. TD: 2240.77 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |