| Economic Development, Investment, Trade & Natural Resources |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Petroleum Branch |  |  |  |  |  |  |  |  |
| 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 |  |  |  |  |  |  |  |  |
| T 204-945-6577 F 204-945-0586 |  |  |  |  |  |  |  |  |
| www.manitoba.ca |  |  |  |  |  |  |  |  |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| (January 1/24 -- November 25/24 ) |  |  |  |  |  |  |  |  |
| DRILLING ACTIVITY |  |  | To Nov 25/24 |  |  | To Nov 27/23 |  | 2023 Total |
| Drilling Licences Issued |  |  | 164 |  |  | 164 |  | 186 |
| Licences Cancelled |  |  | 13 |  |  | 9 |  | 9 |
| Vertical Wells Drilled |  |  | 3 |  |  | 3 |  | 3 |
| Horizontal Wells Drilled |  |  | 177 |  |  | 145 |  | 159 |
| Stratigraphic Test Holes Drilled |  |  | 0 |  |  | 1 |  | 1 |
| Wells Drilled - Total |  |  | 180 |  |  | 149 |  | 163 |
| No. of Metres Drilled |  | 430679 |  |  | 330401 |  |  | 360387 |
| Wells Re-entered |  |  | 1 |  |  | 0 |  | 0 |
| Wells Being Drilled |  |  | 3 |  |  | 2 |  | 0 |
| No. of Active Rigs |  |  | 3 |  |  | 1 |  | 0 |
| Wells Licenced but Not Spudded |  |  | 38 |  |  | 52 |  | 61 |
| Wells Completed as Potential Oil Wells |  |  | 178 |  |  | 146 |  | 160 |
| Wells Abandoned Dry |  |  | 2 |  |  | 3 |  | 3 |
| Wells Drilled but Not Completed |  |  | 0 |  |  | 0 |  | 0 |
| Other Completions |  |  | 1 |  |  | 0 |  | 0 |
| New Wells on Production |  |  | 0 |  |  | 0 |  | 0 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |  |
| Geophysical Programs Licenced |  |  | 2 |  |  | 0 |  | 0 |
| Licences Cancelled |  |  | 0 |  |  | 0 |  | 0 |
| Kilometers Licenced |  |  | 50.31 |  |  | 0.00 |  | 0.00 |
| Kilometers Run |  |  | 50.31 |  |  | 0.00 |  | 0.00 |
|  |  |  | 2024 |  |  | 2023 |  |  |
| OIL PRICES (Average) |  | $/M3 ¦ $/BBL |  |  | $/M3 ¦ $/BBL |  |  |  |
| Month of Septmeber |  | $552.75 | $87.83 |  | $746.94 | $118.69 |  |  |
| Month of October |  | $594.63 | $94.49 |  | $656.24 | $104.28 |  |  |
| OIL PRODUCTION (M3) |  |  | 2024 |  |  | 2023 |  |  |
| Month of October |  | 205,350.6 |  |  | 190,912.6 |  |  |  |

WEEKLY WELL ACTIVITY REPORT

25 November 2024

| Lic. No.: 2335 | Virden Roselea Unit No. 2 WIW 5-7-11-25 (WPM) UWI: 100.05-07-011-25W1.02 Status: Completing (COMP) Completing: 01-Nov-2024 |
| --- | --- |
| Lic. No.: 12115 | Tundra Daly Sinclair HZNTL B4-9-10-29 (WPM) UWI: 103.04-09-010-29W1.00 - Leg #1 Status: Completing (COMP) Completing: 16-Nov-2024 |
| Lic. No.: 12136 | Virden Roselea Unit No.1 HZNTL 10-20-10-25 (WPM) UWI: 102.10-20-010-25W1.00 - Leg #1 Finished Drilling: 22-Nov-2024 Total Depth: 2255.00 m Rig Released: 23-Nov-2024 Surface Casing: 244.50 mm @ 150.00 m with 10.5 t Intermediate Casing: 177.80 mm @ 831.00 m with 16.75 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 23-Nov-2024 |
| Lic. No.: 12168 | North Virden Scallion Unit No. 1 HZNTL 1-22-11-26 (WPM) UWI: 102.01-22-011-26W1.00 - Leg #1 Status: Completing (COMP) Completing: 21-Nov-2024 |
| Lic. No.: 12193 | Virden Roselea Unit No. 1 Prov. WIW HZNTL 7-29-10-25 (WPM) UWI: 102.07-29-010-25W1.00 - Leg #1 Spud Date: 23-Nov-2024 K.B. Elevation: 437.70 m Surface Casing: 244.50 mm @ 150.00 m with 10.5 t Status: Running Casing (CSG) Running Casing: 25-Nov-2024 |
| Lic. No.: 12195 | North Virden Scallion Unit No. 1 WIW HZNTL 5-25-11-26 (WPM) UWI: 102.05-25-011-26W1.00 - Leg #1 Status: Completing (COMP) Completing: 13-Nov-2024 |
| Lic. No.: 12197 | Tundra Daly Sinclair HZNTL C13-21-9-29 (WPM) UWI: 105.13-21-009-29W1.00 - Leg #1 Status: Completing (COMP) Completing: 17-Nov-2024 |
| Lic. No.: 12198 | Tundra Daly Sinclair Prov. HZNTL C4-28-9-29 (WPM) UWI: 104.04-28-009-29W1.00 - Leg #1 Status: Completing (COMP) Completing: 22-Nov-2024 |
| Lic. No.: 12231 | BCX Pierson HZNTL 14-8-3-29 (WPM) UWI: 100.14-08-003-29W1.00 - Leg #1 Status: Completing (COMP) Completing: 18-Nov-2024 |
| Lic. No.: 12236 | Tundra Manson HZNTL 2-8-13-28 (WPM) UWI: 100.02-08-013-28W1.00 - Leg #1 Status: Completing (COMP) Completing: 21-Nov-2024 |
| Lic. No.: 12237 | Tundra Daly Sinclair Prov. HZNTL C9-32-8-29 (WPM) UWI: 104.09-32-008-29W1.00 - Leg #1 Spud Date: 21-Nov-2024 K.B. Elevation: 544.71 m Surface Casing: 244.50 mm @ 144.00 m with 7.0 t |
|  | Status: Running Casing (CSG) Running Casing: 25-Nov-2024 |
| Lic. No.: 12240 | Tundra et al Daly Sinclair HZNTL C13-5-9-28 (WPM) UWI: 104.13-05-009-28W1.00 - Leg #1 Spud Date: 20-Nov-2024 K.B. Elevation: 482.48 m Finished Drilling: 24-Nov-2024 Total Depth: 2402.00 m Rig Released: 25-Nov-2024 Surface Casing: 244.50 mm @ 139.00 m with 8.5 t Production Casing: 139.70 mm @ 2402.00 m with 39.0 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 25-Nov-2024 |
| Lic. No.: 12241 | Tundra et al Daly Sinclair HZNTL C12-5-9-28 (WPM) UWI: 104.12-05-009-28W1.00 - Leg #1 Finished Drilling: 18-Nov-2024 Total Depth: 2387.00 m Rig Released: 19-Nov-2024 Surface Casing: 244.50 mm @ 143.00 m with 8.5 t Production Casing: 139.70 mm @ 2387.00 m with 37.5 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 19-Nov-2024 |
| Lic. No.: 12242 | Sinclair Unit No. 19 HZNTL C5-9-9-29 (WPM) UWI: 104.05-09-009-29W1.00 - Leg #1 Spud Date: 24-Nov-2024 K.B. Elevation: 535.25 m Surface Casing: 244.50 mm @ 137.00 m with 7.5 t |
|  | Status: Waiting On Cement (WOC) Waiting On Cement: 25-Nov-2024 |
| Lic. No.: 12243 | Tundra Daly Sinclair Prov. HZNTL A1-32-9-29 (WPM) UWI: 103.01-32-009-29W1.00 - Leg #1 Spud Date: 18-Nov-2024 K.B. Elevation: 544.92 m Finished Drilling: 23-Nov-2024 Total Depth: 2397.00 m Rig Released: 24-Nov-2024 Surface Casing: 244.50 mm @ 142.00 m with 7.5 t Production Casing: 139.70 mm @ 2397.00 m with 40.05 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 24-Nov-2024 |
| Lic. No.: 12248 | Tundra Daly Sinclair HZNTL B1-8-10-29 (WPM) UWI:103.01-08-010-29W1.00 Tundra Daly Sinclair HZNTL B01-08-10-29 (WPM) Licence Issued: 22-Nov-2024 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 8A-7-10-29 Co-ords: 418.05 m N of S of Sec 7 60.00 m W of E of Sec 7 Grd Elev: 530.71 m Proj. TD: 2352.56 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
| Lic. No.: 12249 | Virden Roselea Unit No. 1 Prov. HZNTL 2-10-10-26 (WPM) UWI:102.02-10-010-26W1.00 Virden Roselea Unit No. 1 Prov. HZNTL 02-10-10-26 (WPM) Licence Issued: 22-Nov-2024 Licensee: Corex Resources Ltd Mineral Rights: Corex Resources Ltd Contractor: Ensign Drilling Inc. - Rig# 14 Surface Location: 3C-11-10-26 Co-ords: 227.39 m N of S of Sec 11 521.38 m E of W of Sec 11 Grd Elev: 434.79 m Proj. TD: 1850.25 m (Mississippian) Field: Virden Classification: Non Confidential Development Status: Location(LOC) |

WELL SUSPENSIONS

| Lic. No. | Well Name and Location | Suspension & Expiry Date | Status |
| --- | --- | --- | --- |
| 4037 | South Pierson Unit No. 1 Prov. WIW 14-4-2-29 (WPM) UWI: 100.14-04-002-29W1.00 | 19-Nov-2024 03-Jan-2025 | WIW |
| 6478 | Tundra Regent HZNTL 13-13-4-22 (WPM) UWI: 100.04-24-004-22W1.02 UWI: 100.13-13-004-22W1.00 | 19-Nov-2024 03-Jan-2025 | COOP |

WELLS RELEASED OFF CONFIDENTIAL

The following wells were released from confidential during the last week.  To obtain well or production data for these wells, please contact Paulette Seymour at (204) 945-6575.

| Lic. No. | Well Name and Location | Field/Pool Code |
| --- | --- | --- |
| 12123 | North Virden Scallion Unit No. 1 WIW A7-16-11-26 (WPM) | 5 |
| 12170 | Tundra Pierson HZNTL 6-12-3-29 (WPM) | 7 |
| 12172 | Corex Virden HZNTL A10-17-11-26 (WPM) | 5 |
| 12204 | Ewart Unit No. 14 Prov. HZNTL A11-2-8-28 (WPM) | 1 |
| 12208 | Corex Daly Sinclair HZNTL C4-4-10-28 (WPM) | 1 |
| 12210 | Melita Pierson HZNTL 16-28-2-28 (WPM) | 7 |
| 12218 | Tundra Virden HZNTL 8-31-10-26 (WPM) | 5 |
| 12219 | Tundra Virden HZNTL A12-21-11-26 (WPM) | 5 |
| 12221 | Tundra Daly Sinclair HZNTL A9-36-9-28 (WPM) | 1 |
| 12222 | Tundra Daly Sinclair HZNTL B16-36-9-28 (WPM) | 1 |
| 12223 | Tundra Virden HZNTL 9-27-10-26 (WPM) | 5 |
| 12224 | Tundra Pierson HZNTL A12-19-1-28 (WPM) | 7 |