| Economic Development, Investment, Trade & Natural Resources |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Petroleum Branch |  |  |  |  |  |  |  |  |
| 360-1395 Ellice Ave, Winnipeg, MB R3G 3P2 |  |  |  |  |  |  |  |  |
| T 204-945-6577 F 204-945-0586 |  |  |  |  |  |  |  |  |
| www.manitoba.ca |  |  |  |  |  |  |  |  |
| WEEKLY WELL ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| PETROLEUM INDUSTRY ACTIVITY REPORT |  |  |  |  |  |  |  |  |
| (January 1/24 -- December 02/24 ) |  |  |  |  |  |  |  |  |
| DRILLING ACTIVITY |  |  | To Dec 02/24 |  |  | To Dec 04/23 |  | 2023 Total |
| Drilling Licences Issued |  |  | 166 |  |  | 170 |  | 186 |
| Licences Cancelled |  |  | 13 |  |  | 9 |  | 9 |
| Vertical Wells Drilled |  |  | 3 |  |  | 3 |  | 3 |
| Horizontal Wells Drilled |  |  | 183 |  |  | 150 |  | 159 |
| Stratigraphic Test Holes Drilled |  |  | 0 |  |  | 1 |  | 1 |
| Wells Drilled - Total |  |  | 186 |  |  | 154 |  | 163 |
| No. of Metres Drilled |  | 446047 |  |  | 344748 |  |  | 360387 |
| Wells Re-entered |  |  | 1 |  |  | 0 |  | 0 |
| Wells Being Drilled |  |  | 2 |  |  | 2 |  | 0 |
| No. of Active Rigs |  |  | 2 |  |  | 2 |  | 0 |
| Wells Licenced but Not Spudded |  |  | 35 |  |  | 52 |  | 61 |
| Wells Completed as Potential Oil Wells |  |  | 184 |  |  | 151 |  | 160 |
| Wells Abandoned Dry |  |  | 2 |  |  | 3 |  | 3 |
| Wells Drilled but Not Completed |  |  | 0 |  |  | 0 |  | 0 |
| Other Completions |  |  | 1 |  |  | 0 |  | 0 |
| New Wells on Production |  |  | 0 |  |  | 0 |  | 0 |
| GEOPHYSICAL ACTIVITY |  |  |  |  |  |  |  |  |
| Geophysical Programs Licenced |  |  | 2 |  |  | 0 |  | 0 |
| Licences Cancelled |  |  | 0 |  |  | 0 |  | 0 |
| Kilometers Licenced |  |  | 50.31 |  |  | 0.00 |  | 0.00 |
| Kilometers Run |  |  | 50.31 |  |  | 0.00 |  | 0.00 |
|  |  |  | 2024 |  |  | 2023 |  |  |
| OIL PRICES (Average) |  | $/M3 ¦ $/BBL |  |  | $/M3 ¦ $/BBL |  |  |  |
| Month of October |  | $594.63 | $94.49 |  | $656.24 | $104.28 |  |  |
| Month of November |  | $588.81 | $93.56 |  | $586.54 | $93.20 |  |  |
| OIL PRODUCTION (M3) |  |  | 2024 |  |  | 2023 |  |  |
| Month of November |  | 199,661.4 |  |  | 188,028.0 |  |  |  |

WEEKLY WELL ACTIVITY REPORT

02 December 2024

| Lic. No.: 10815 | Sinclair Unit No. 1 Prov. OBS A10-4-8-29 (WPM) UWI: 102.10-04-008-29W1.00 |
| --- | --- |
|  | Status: Completing (COMP) Completing: 10-Nov-2024 |
| Lic. No.: 12171 | Virden Roselea Unit No. 3 HZNTL A3-23-10-26 (WPM) UWI: 103.03-23-010-26W1.00 - Leg #1 Spud Date: 29-Nov-2024 K.B. Elevation: 441.83 m Surface Casing: 244.50 mm @ 150.00 m with 10.5 t Intermediate Casing: 177.80 mm @ 831.00 m with 16.5 t Status: Drilling Ahead (DR) Drilling Ahead: 02-Dec-2024 |
| Lic. No.: 12173 | Tundra Daly Sinclair HZNTL 4-31-9-29 (WPM) UWI: 100.04-31-009-29W1.00 - Leg #1 Status: Completing (COMP) Completing: 26-Nov-2024 |
| Lic. No.: 12193 | Virden Roselea Unit No. 1 Prov. WIW HZNTL 7-29-10-25 (WPM) UWI: 102.07-29-010-25W1.00 - Leg #1 Finished Drilling: 28-Nov-2024 Total Depth: 2743.00 m Rig Released: 28-Nov-2024 Surface Casing: 244.50 mm @ 150.00 m with 10.5 t Intermediate Casing: 177.80 mm @ 726.00 m with 15.0 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 28-Nov-2024 |
|  | Status: Completing (COMP) Completing: 02-Dec-2024 |
| Lic. No.: 12232 | North Virden Scallion Unit No. 1 WIW HZNTL A1-22-11-26 (WPM) UWI: 103.01-22-011-26W1.00 - Leg #1 Status: Completing (COMP) Completing: 27-Nov-2024 |
| Lic. No.: 12237 | Tundra Daly Sinclair Prov. HZNTL C9-32-8-29 (WPM) UWI: 104.09-32-008-29W1.00 - Leg #1 Finished Drilling: 24-Nov-2024 Total Depth: 2425.00 m Rig Released: 25-Nov-2024 Surface Casing: 244.50 mm @ 144.00 m with 7.0 t Production Casing: 139.70 mm @ 2425.00 m with 39.0 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 26-Nov-2024 |
| Lic. No.: 12239 | Tundra East Manson HZNTL 3-9-13-28 (WPM) UWI: 100.03-09-013-28W1.00 - Leg #1 Status: Completing (COMP) Completing: 25-Nov-2024 |
| Lic. No.: 12242 | Sinclair Unit No. 19 HZNTL C5-9-9-29 (WPM) UWI: 104.05-09-009-29W1.00 - Leg #1 Finished Drilling: 27-Nov-2024 Total Depth: 2424.00 m Rig Released: 28-Nov-2024 Surface Casing: 244.50 mm @ 137.00 m with 7.5 t Production Casing: 139.70 mm @ 2424.00 m with 40.5 t |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 28-Nov-2024 |
| Lic. No.: 12244 | Tundra Daly Sinclair Prov. HZNTL 13-10-11-29 (WPM) UWI: 102.13-10-011-29W1.00 - Leg #1 Spud Date: 26-Nov-2024 K.B. Elevation: 531.98 m Finished Drilling: 30-Nov-2024 Total Depth: 2307.00 m Rig Released: 01-Dec-2024 Surface Casing: 244.50 mm @ 156.00 m with 7.0 t Production Casing: 139.70 mm @ 2307.00 m with 36.5 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 01-Dec-2024 |
| Lic. No.: 12245 | Tundra et al Daly Sinclair HZNTL C9-28-8-28 (WPM) UWI: 104.09-28-008-28W1.00 - Leg #1 Spud Date: 28-Nov-2024 K.B. Elevation: 499.16 m Finished Drilling: 01-Dec-2024 Total Depth: 2132.00 m Rig Released: 02-Dec-2024 Surface Casing: 244.50 mm @ 135.00 m with 7.5 t Production Casing: 139.70 mm @ 2132.00 m with 25.09 t |
|  | Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 02-Dec-2024 |
| Lic. No.: 12247 | Tundra Pierson Prov. HZNTL 3-22-2-28 (WPM) UWI: 100.03-22-002-28W1.00 - Leg #1 Spud Date: 25-Nov-2024 K.B. Elevation: 461.42 m Finished Drilling: 30-Nov-2024 Total Depth: 3337.00 m Rig Released: 01-Dec-2024 Surface Casing: 219.10 mm @ 141.00 m with 7.0 t Production Casing: 139.70 mm @ 3337.00 m with 44.0 t Status: Waiting On Service Rig (WOSR) Waiting On Service Rig: 01-Dec-2024 |
| Lic. No.: 12250 | Tundra Daly Sinclair HZNTL 9-15-10-28 (WPM) UWI: 102.09-15-010-28W1.00 - Leg #1 Spud Date: 01-Dec-2024 K.B. Elevation: 514.03 m Surface Casing: 244.50 mm @ 140.00 m with 7.0 t |
|  | Tundra Daly Sinclair HZNTL 09-15-10-28 (WPM) Licence Issued: 28-Nov-2024 Licensee: Tundra Mineral Rights: Tundra Contractor: Ensign Drilling Inc. - Rig# 12 Surface Location: 9D-16-10-28 Co-ords: 578.83 m S of N of Sec 16 60.00 m W of E of Sec 16 Grd Elev: 511.20 m Proj. TD: 2279.85 m (Mississippian) Field: Daly Sinclair Classification: Non Confidential Development Status: Location(LOC) |
|  | Status: Waiting On Cement (WOC) Waiting On Cement: 02-Dec-2024 |
| Lic. No.: 12251 | Tundra Pierson Prov. HZNTL 6-11-2-28 (WPM) UWI:100.06-11-002-28W1.00 Tundra Pierson Prov. HZNTL 06-11-02-28 (WPM) Licence Issued: 28-Nov-2024 Licensee: Tundra Oil & Gas Limited Mineral Rights: Tundra Oil & Gas Limited Contractor: Ensign Drilling Inc. - Rig# 10 Surface Location: 8A-9-2-28 Co-ords: 517.04 m N of S of Sec 9 60.00 m W of E of Sec 9 Grd Elev: 458.96 m Proj. TD: 3334.49 m (Triassic) Field: Pierson Classification: Non Confidential Development Status: Location(LOC) |

WELLS RELEASED OFF CONFIDENTIAL

The following wells were released from confidential during the last week.  To obtain well or production data for these wells, please contact Paulette Seymour at (204) 945-6575.

| Lic. No. | Well Name and Location | Field/Pool Code |
| --- | --- | --- |
| 12170 | Tundra Pierson HZNTL 6-12-3-29 (WPM) | 7 |
| 12172 | Corex Virden HZNTL A10-17-11-26 (WPM) | 5 |
| 12210 | Melita Pierson HZNTL 16-28-2-28 (WPM) | 7 |
| 12221 | Tundra Daly Sinclair HZNTL A9-36-9-28 (WPM) | 1 |
| 12223 | Tundra Virden HZNTL 9-27-10-26 (WPM) | 5 |